Control and Management of Audio/Video Streams

OMG RFP Submission

Revised Submission

telecom/98-10-05

IONA Technologies, Plc

Lucent Technologies, Inc

Siemens-Nixdorf, AG

Supported by

Fore Systems, Inc

Version 1.2

October, 1997

Copyright 1997	IONA Technologies, Plc.; Lucent Technologies, Inc; Siemens-Nixdorf, AG.

All Rights Reserved.

The companies and organisations listed above hereby grant a royalty-free license to the Object Management Group (OMG) for world-wide distribution of this document or any derivative works thereof, so long as the OMG reproduces the copyright notices and the below paragraphs on all distributed copies.

The material in this document is provided for evaluation by the OMG. Submission of this document does not represent a commitment to implement any portion of this specification in the products of the above said companies and organisations.

While the information in the publication is believed to be accurate, the companies and organisations listed above make no warranty of any kind with regard to this material including but not limited to the implied warranties of merchantability and fitness for a particular purpose. The companies and organisations listed above shall not be liable for errors contained herein or for incidental or consequent damages in connection with the furnishing, performance or use of this material. The information contained in this document is subject to change without notice.

This document contains information which is protected by copyright. All Rights Reserved. No part of this work covered by copyright here on may be reproduced or used in any form or by any means – graphic, electronic, or mechanical, including photocopying, recording, taping, or information storage and retrieval systems – without permission of the copyright owners. All copies of this document must include the copyright and other information contained on this page.

The copyright owners grant the Object Management Group permission to reproduce and use the information contained in this document. The copyright owners grant a limited waiver of the copyright on this document to members of the Object Management Group so that they can each reproduce up to 50 copies of this document for their internal use as part of the OMG evaluation process.

RIGHTS RESERVED LEGEND. Use, duplication, or disclosure by government is subject to restrictions as set forth in subdivision (c)(1)(ii) of the Right in Technical Data and Computer Software Clause at DFARS 252.227.7013.

All trademarks acknowledged.

1. Introduction

2. Architecture Overview

2.1 (Virtual) Multimedia Device Interface

2.2 StreamCtrl Interface

2.3 StreamEndPoint Interface

2.4 Flow Endpoints, Flow Connections and FDevs

2.5 Properties of Streams

2.6 Quality of Service

2.7 Stream specification

2.8 Flow Protocol

2.9 Examples for point-to-point streams

2.10 Issues in Modifying QoS

2.11 Issues in Multipoint Streams

2.12 Extending Stream Management Functionality

2.13 Device and Stream Parameters

3. IDL Interfaces

3.1 The Basic_StreamCtrl

3.2 The StreamCtrl

3.3 The StreamEndpoint

3.3.1 connect() and request_connection()

3.3.2 Other Functions Supported by StreamEndPoint

3.4 The StreamEndPoint_A and StreamEndPoint_B

3.4.1 Multicasting Operations

3.4.2 Specialisations of StreamEndPoint A and B

3.5 The MMDevice

3.6 The VDev

3.7 The FlowEndPoint

3.8 The FlowConnection

3.9 FDev

4. Conformance Criteria

4.1 Light vs Full Profile

4.2 Flow Protocol

4.3 Network QoS Parameters

5. Appendix A - Meeting RFP Requirements for Control and Management of A/V Streams

5.1 Topologies

5.2 Multiple Flows

5.3 Stream Description and Typing

5.4 Stream Interface Identification and Reference

5.5 Stream setup and release

5.6 Stream Modification

5.7 Stream Termination

5.8 Multiple Protocols

5.9 Flow Synchronisation

5.10 Interoperability

5.10.1 Naming of Generated Interfaces

5.10.2 Compatibility of StreamEndPoints

5.10.3 Compatibility of Flow Formats

5.11 Security

5.12 The A/V Streams Registration Space

5.12.1 Carrier Protocol Names

5.12.2 SFP

5.12.3 Media Format Categories and MIME Content-Types

5.12.4 QoS Parameters

5.12.5 Device Parameters

6. Appendix B - Full IDL

7. Appendix C - Relationship to DAVIC work

8. References

Submission Contact Points

All questions regarding this joint submission should be directed to:

Damian McGrath (Editor) IONA Technologies Plc 8-10 Lower Pembroke Street Dublin 2 Ireland telephone: +353 1 662 5255 facsimile: +353 1 662 5244 email: dmcgrath@iona.com

Tom Rutt Lucent Technologies Bell Labs - Global Strategic Standards Rm 4L-408 101 Crawford Corner Rd. Holmdel, NJ 07733 USA phone +1 908 949 7862 fax +1 908 949 1196 E-mail: t.rutt@lucent.com

Jörg Ottensmeyer Siemens AG, ZT IK 2 Otto-Hahn-Ring 6 81730 München Germany email: Joerg.Ottensmeyer@mchp.siemens.de

Trade Marks

All trademarks acknowledged.

Acknowledgements

The submitters would like to thank Cosmos Nicolaou and his colleagues of Fore Systems, Inc. Audio/Video Systems for their support and input into this RFP response. Their ideas have greatly influenced the shape of the solution. Many other parties have also influenced this RFP response. The submitters would like, in particular, to thank Chris Smith of BT for his input to the VDev negotiation process and Ross Mayne of Broadcom Eireann Research for his suggested use of MIME content types. Thanks are due also to the TINA Core Team for their review comments.

Some of the concepts in this submission were inspired by the IMA MSS specification.

Notes

Throughout this document the article ‘he’ is used generically to refer to a man or a woman.

# Introduction

IONA Technologies, Plc,  Lucent Technologies, Inc. and Siemens-Nixdorf, AG are pleased to offer the following revised submission to the OMG’s Control and Management of A/V Streams RFP (telecom/96-09-01).

The Streams RFP contains 12 issues to be addressed:

- Topologies for streams

- Multiple flows

- Stream description and typing

- Stream interface identification and reference

- Stream set-up and release

- Stream modification

- Stream termination

- Multiple protocols

- Quality of Service

- Flow Synchronisation

- Interoperability

- Security

This revised RFP response addresses issues 1-11 and provides hooks for solutions to issue 12.

The example stream architecture from the OMG “Control of A/V Streams” RFP is depicted in Figure 1.  This RFP submission uses an architecture based upon Figure 1.  The term Stream control RFP is used throughout this submission as an abbreviation for the name of the RFP.

Figure 1 - Example stream architecture from the RFP

Figure 1 shows a stream with a single flow between two stream end-points, one acting as the source of the data and the other the sink. Each stream end-point, shown as a dotted encapsulation in Figure 1, consists of three logical entities: a stream interface control object, that provides IDL defined interfaces (as server, 2b) for controlling and managing the stream (as well as potentially, outside the scope of this submission, invoking operations as client, 2a, on other server objects), a flow data source or sink object (at least one per stream endpoint)  that is the final destination of the data flow (3), and a stream adaptor that transmits and receives frames over a network.

The Stream interface control object is shown in Figure 1 as using a basic object adaptor, that transmits and receives control messages in a CORBA compliant way.  This RFP submission does not require any changes to the BOA/POA or IDL language bindings to accommodate the control of A/V streams.

When a stream is terminated in hardware, the source/sink object and the stream adaptor may not be visible as distinct entities. How the stream interface control object communicates with the source/sink object and perhaps indirectly with the stream adaptor (interface 4 in Figure1.), and how the source/sink object communicates with the stream adaptor (interface 3 in Figure 1.) are out-side the scope of this RFP submission.

This “Control of A/V Streams” RFP submission provides definitions of the components that make up a stream and for interface definitions onto Stream control and management objects (interface number 1a.), and for interface definitions onto stream interface control objects (interface number 2b.) associated with individual stream endpoints.

In particular, CORBA interface references for stream interface control objects are used to refer to all stream endpoints in parameters to stream control operations defined in this submission.  Thus, this submission does not need to define a new IDL data type for stream interface reference.

# Architecture Overview

This document proposes a set of interfaces which implement a distributed media streaming framework. The principal components of the framework are:

- Virtual Multimedia Devices and Multimedia device - represented by the VDev and MMDevice interfaces respectively

- Streams - represented by the StreamCtrl interface

- Stream endpoints - represented by the StreamEndPoint interfaces

- Flows and flow endpoints - represented by FlowConnection and FlowEndPoint interfaces respectively

- Flow Devices - Represented by the FDev interface.

A stream represents continuous media transfer, usually between two or more virtual multimedia devices. A stream endpoint terminates a stream. A simple stream between a microphone device (audio source or producer) and speaker device (audio sink or consumer) is shown in Figure 2.

Figure 2 - A basic stream configuration

A stream may contain multiple flows. Each flow carries data in one direction so a flow endpoint may be either a source (producer) or a sink (consumer). An operation on a stream (for example, stop or start) may be applied to all flows within the stream simultaneously or just a subset of them.

A stream endpoint may contain multiple flow endpoints. Both flow producer endpoints and flow consumer endpoints may be contained in the same stream endpoint. There may be a CORBA object representing each flow endpoint and flow connection (i.e. the flow itself), but not all systems are required to expose IDL interfaces to these flow objects. Figure 3 illustrates a stream which consists of several different flow connections.  Note that not all flow endpoints are involved in the stream, i.e. there may be ‘dangling’ flow endpoints. Note also that flows can travel in both directions within the same stream. When two stream endpoints which support separate flow endpoints are bound, a compatability rule can be used to determine which flow endpoints connect to eachother.

Figure 3 -  stream connection compatibility rules can allow unconnected flow endpoints

A multimedia device abstracts one or more items of multimedia hardware and acts as a factory for virtual multimedia devices. A multimedia device can support more than one stream simultaneously, for example a microphone device streaming audio to two speaker devices using separate non-multicast connections. For each stream connection requested, the multimedia device creates a stream endpoint and a virtual multimedia device.

The StreamEndPoint interface type has two specialisations StreamEndPoint_A and StreamEndPoint_B. This does not imply that the flows always start at the A party and flow to the B party, indeed both A and B parties may have a mixture of flow producers and flow consumers. Stream endpoints are distinguished in this way in order to help the implementation determine the directionality of their contained flow endpoints. For example, a videophone stream may contain four flows labelled video1, video2, audio1, audio2. When a videophone A party endpoint is created it can automatically set  video1 to be a consumer, video2 to be a producer and so on. Similarly, when the videophone B party endpoint is created it can set video1 to be a producer and video2 to be a consumer and so on. In this way the videophone A and B parties can act like a ‘plug and socket’ with all pins and ports facing in the right direction, relieving the application programmer of the chore of manually ensuring that this is the case.

Throughout this document the verb ‘binding’ when applied to a stream, flow or set of multimedia devices  is used as meaning stream or flow establishment. When ‘binding’ is used as a noun it refers to a stream or flow which is established i.e. has active transport connections.

The sections below discuss each of the main IDL interfaces in more detail. There are two basic ‘profiles’ for the streaming service:

- The ‘full profile’ in which flows endpoints and flow connections have accessable IDL interfaces. This profile is optimised for flexibility.

- The ‘light’ profile in which flows endpoints and flow connections do not expose IDL interfaces. The light profile is a subset of the full profile. It is optimised for systems which need to minimise memory footprint and the number of CORBA invocations.

## (Virtual) Multimedia Device Interface

A multimedia device abstracts one or more items of multimedia hardware and is described by the IDL interface MMDevice. A multimedia device can be connected or ‘bound’ to one or more compatible multimedia devices using a stream. A multimedia device can potentially support any number of streams to other multimedia devices. Each stream connection will be supported by creation of a virtual multimedia device (VDev) and a stream endpoint (StreamEndPoint) representing the device specific and network specific aspects of a stream endpoint respectively. Virtual devices will have configuration parameters associated with them. For example, a microphone device might be capable of encoding audio data using either m-law or A-law. The sampling frequency might also vary. When two virtual devices are connected using a stream they must ensure that they are both appropriately configured. For example, if the microphone device is using A-law encoding and a sampling frequency of 8-kHz then it must ensure that the speaker device it is connected to is similarly configured. It does this by calling configuration operations such as set_format(“audio1”,”MIME:audio/basic”) and

set_dev_params(“audio1”,myaudioProperties)

on the VDev interface for the speaker device. This procedure occurs when a StreamCtrl is establishing a stream between two virtual devices. Changes in configuration can also occur while a stream is flowing between two virtual devices. There are some important points to note about this procedure. The VDev negotiation procedure is initiated in a point-to-point scenario by the streamCtrl first calling set_peer() on the ‘A-party’ VDev with the ‘B-party’ VDev as a parameter and then calling set_peer() on the ‘B-party’ VDev with the ‘A-party’ VDev as a parameter. It will generally be the case that the programmer who implements the VDev (either an application programmer or a multimedia device vendor) will be responsible for writing the behaviour of the set_peer() call. This behaviour should normally consist of checking that all flows which originate in this VDev can be understood by the corresponding flow consumers in the peer VDev. The operations set_format() and set_dev_params() have been expressly included for this purpose. A variation on this procedure is also used for multipoint streams and will be explained in Section 3.2.

An MMDevice supports operations to set up a stream between two or more peer MMDevices on the network. For example, to bind two multimedia devices using a stream the application programmer could call the following operation on the MMDevice interface:

StreamCtrl bind(in MMDevice peer_device, inout streamQoS the_qos,

out boolean is_met, flowSpec the_spec);

The parameter peer_device specifies the device which should be bound to this device using the stream. The the_spec parameter is a list which specifies the flows within the stream which should be initially connected. If the list is empty then all flows are connected. This operation will return a reference to the StreamCtrl which the application programmer can then use to stop, start or otherwise manipulate the stream. This StreamCtrl will be co-located with the MMDevice on which the bind call is made, it can therefore be considered a form of first-party stream establishment.

## StreamCtrl Interface

The StreamCtrl interface abstracts a continuous media transfer between virtual devices. It supports operations to bind multimedia devices using a stream. There are also operations to start and stop a stream. If the application programmer requires more complex functionality (for example, rewind) he can extend the basic stream interface to support this.

When the application programmer requests a stream between two or more multimedia devices he can explicitly specify the Quality of Service (QoS) of the stream. QoS can mean different things at different levels. For example, a multimedia device which supports video will be concerned with QoS parameters like frame-rate and colour depth. This type of QoS will be referred to as application-level QoS. A stream, however, will be supported by underlying network protocols. The QoS for a network protocol may include parameters like minimum bandwidth and jitter. This type of QoS will be referred to as network-level QoS. An application programmer can establish a stream between two devices by calling the operation bind_devs() on the StreamCtrl interface:

boolean bind_devs(in MMDevice a_party, in MMDevice b_party,

inout streamQoS the_qos, flowSpec the_spec);

The StreamCtrl object is generally instantiated by the application programmer (see section 2.9). The bind() operation on the MMDevice interface is simply a shorthand for creating a StreamCtrl and calling bind_devs() on it. The argument the_qos will generally be an application-level QoS specification. The appropriate protocols will be chosen by the StreamCtrl and a translation will occur between application level QoS and network level QoS for these protocols. If the MMDevices decide they can accept the connection, they will each create a StreamEndPoint and a VDev to support the stream between them.

The Media Streaming Framework supports point-to-multipoint streams. These allow a single ‘source’ device to be connected to many sink devices via a multipoint stream. For example the application programmer could do something like the following to set up a video broadcast:

MyStream->bind_devs(cameraDev,nilObject,someQoS,nilFlowSpec)

Using a reference to a nil Object as the b_party parameter has the effect of adding a camera device as a multicast A party. The application programmer can then add an arbitrary number of TVs to the broadcast by calling

MyStream->bind_devs(nilObject,TVDev,someQoS,nilFlowSpec)

Note that in the point-to-point case, an A party may contain flow sinks and the B party the corresponding flow sources but in the multipoint case only flow sources may be present in an A-party and only flow sinks in a B-party. The A and B stream endpoints are therefore symmetrical except for the multipoint case.

## StreamEndPoint Interface

A stream endpoint logically contains and controls the flow endpoints for each of the individual flows in a stream. There are two ‘flavours’ of stream endpoint: an ‘A-party’ and a ‘B-party’ (represented by the interfaces StreamEndPoint_A and StreamEndPoint_B). An A party can contain producer flow endpoints as well as consumer flow endpoints, similarly with a B party. The primary reason for making a distinction is so that when an instance of a typed StreamEndPoint is created the flows will be plumbed in the right direction i.e. An audio consumer FlowEndPoint in a StreamEndPoint_A will correspond to an audio producer FlowEndPoint in a StreamEndPoint_B. The choice of which end-point is the A party and which is the B party is entirely arbitrary. A call to connect() which will establish or ‘bind’ the stream can be made on either a StreamEndPoint_A or a StreamEndPoint_B. Figure 4 illustrates what happens when the bind_devs() operation is called on a StreamCtrl object.

Figure 4 - Establishing a stream (simplified)

Step 1: A StreamCtrl object may be created by the application programmer to initiate a stream between two multimedia devices. To establish a stream between aMMDev and bMMDev, a call is made to bind_devs() on the StreamCtrl interface.

Step 2: The StreamCtrl asks aMMDev and bMMDev for a StreamEndPoint and VDev to support the stream by calling create_A(…,someQoS,…) on one and create_B(…,someQoS,…) on the other. The former call, for example, will return a StreamEndPoint_A object which is associated with aVDev. This is the point at which an MMDevice could decide that it can support no more connections and refuse to create the StreamEndPoint and VDev. A particular MMDevice might be specialised to only create A-end-points for a type of stream (for example, a microphone end-point and VDev) or only B-end-points (for example, a speaker end-point and VDev).

Step 3: This step involves the aVDev calling configuration operations on the bVDev and vice versa. This is the device configuration phase already discussed under the Multimedia Devices heading.

Step 4: The actual stream is set up by calling connect() on the A_EndPoint with the B_EndPoint as a parameter (or alternatively call connect() on the B_EndPoint with the A_EndPoint as a parameter). Stream end-points contain a number of flow end-point objects. A flow end-point object  can be used to either pull information from a multimedia device and send it over the network or vice versa. In the light profile the FlowEndPoint objects are co-located with the StreamEndPoint objects and do not expose IDL interfaces.

Step 5: The A_EndPoint may choose to listen on transport addresses for a subset of the flows to be connected. It will then call request_connection(). Among the information passed across will be the transport addresses of the listening flows on the A side. The B_EndPoint will connect to the listening flows on the A-side and will listen on transport addresses for any remaining flows for which the A side is not listening. Among  the information passed back by the request_connection() operation will be the transport addresses of the flows listening on the B side. The final stage is for the A_EndPoint to connect to the listening flows on the B_EndPoint.

So far, streams have always been shown between multimedia devices. It is possible that a programmer will want to stream data between objects in a distributed application which have nothing to do with multimedia devices. For example, streaming updates of the positions of players in a distributed multi-player game. Where there is no analogue of a multimedia device involved then asking the application programmer to specify a ‘dummy’ multimedia device class is inappropriate and an un-necessary overhead. The framework makes it possible to have a stream existing independently of any multimedia devices. The stream will be established directly between two StreamEndPoints which have been created by the application programmer. This can be done by making the following call on the StreamCtrl interface:

boolean bind(in StreamEndPoint_A A_party,

in StreamEndPoint_B B_party,

inout streamQoS theQoS,

in flowSpec theFlows)

The bind() call can also be used to set up multipoint streams in exactly the same way as a bind_devs() call.

## Flow Endpoints, Flow Connections and FDevs

In the above example for stream setup, the stream the light profile was assumed in which the flow endpoints are co-located with the stream endpoints. In systems which support the ‘full’ as opposed to the ‘light’ streams profile, individual flow endpoints and flow connections (which connect flow endpoints) are accessible. This gives greater granularity of control over stream establishment and manipulation. Indeed, a stream can be established by simply setting up a set of FlowConnections and then grouping them under a StreamCtrl.

A flow endpoint supports an operation is_fep_compatible(). This allows a third party (usually a FlowConnection) to check whether two flow endpoints are connectable. A flow endpoint may support a selection of protocols and a selection of encoding/decoding formats. Two flow endpoints are compatible if they share a common protocol over which they can transport media, and if they share a common format for media encoding/decoding (e.g. MPEG). The FDev is exactly analogous to the MMDevice for streams. A FlowConnection can be used to bind FDevs in exactly the same manner as a StreamCtrl binds MMDevices. Whereas an MMDevice creates a StreamEndPoint and a VDev, an FDev just creates a FlowEndPoint. The FlowEndPoint combines the functionality of a VDev and a StreamEndPoint in a single interface but only applies to a single flow.

The algorithm for connecting a set of flow endpoints can then be specialised for supporting a particular type of stream connection. For example, in Figure 3 the top dangling flow endpoint in stream endpoint A might be a French language audio producer, while the next lowest flow endpoint might be the English language audio producer, and the other two might be video provider and talk-back audio consumer flow endpoints.  Stream endpoint A is compatible with stream endpoint C, even though stream endpoint C has no French audio consumer. In a given request for a stream connection (or connection modification), a subset of flow endpoints may be selected.

In general, the compatibility rules for connecting two stream endpoints (A and C) are as follows:

- The stream endpoint types for both A and C must specify which flow endpoint types are included, along with the causality (producer or consumer) of each flow endpoint.

- When stream connection setup is requested, the client may select a subset of the flow endpoints to be included in the stream connection for a specified stream endpoint.  For each flow endpoint selected in each stream endpoint (default is to try to connect all flow endpoints), the connection provided must determine if there is a compatible flow endpoint in the other stream endpoint using the flow compatibility rules outlined above.

- The two stream endpoints are compatible if  they both contain at least one flow endpoint which is compatible with at-least one flow endpoint in the other stream endpoint.  In the case of one-to-many flow endpoint compatibility choices, a best fit algorithm could be used to select one-to-one associations of flow endpoints in the two stream endpoints.  Flow endpoints in each stream endpoint, which have no compatible flow endpoints in the other stream endpoints are called “dangling” flow endpoints, and cannot participate in a stream connection involving those two stream endpoints.

- In order to set-up a stream connection there must be at-least one pair of compatible flow endpoints in the connected stream endpoints.  If this is not so, the stream connection will fail.

- A stream connection may be modified by requesting additional flow endpoints (included in the stream endpoint type) be added or dropped from an existing connection.  In such cases, the stream controller attempts to connect the newly selected flow endpoints with an associated flow endpoint on the other side of the stream connection.

If a Media Streams implementation supports the full profile of operation, then making a bind call on StreamCtrl leads to the execution of the certain behaviour for establishing whether the stream setup is possible. First consider the scenario where the StreamCtrl has been implemented by a vendor who has supplied the full profile, the StreamEndPoint_A is running on a platform which is supplied with a full profile implementation and StreamEndPoint_B is from a light profile implementation. Because the StreamCtrl is a full profile implementation, it will attempt to bind the stream using the algorithm above. It will begin by querying both StreamEndPoints to see what flows they support SEP_A->get_property_value(“Flows”) and SEP_B->get_property_value(“Flows”). It will then attempt to iterate through the flows on A and B by calling an operation get_fep(“flowName”) for each flow. On the StreamEndPoint_A (which is full profile) this will return an Object which can be narrowed to a FlowEndPoint but on the StreamEndPoint_B (light profile) this operation will raise the exception notSupported with reason “Full profile not supported”. The StreamCtrl will then realise that this StreamEndPoint is a light profile one so it will proceed by using the light profile algorithm for stream establishment, i.e. it calls connect() on StreamEndPoint_A with StreamEndPoint_B as a parameter (In fact, the connect() operation works just as well from a B endpoint to an A endpoint as it does from an A to a B).

If on the other hand, both StreamEndPoints were both full profile then the get_fep() operation would succeed on both and the StreamCtrl would build a list of FlowEndPoints on the A side and on the B side. It will use the algorithm above to iterate through these lists matching FlowEndPoints on the A side to FlowEndPoints on the B side. Once it is established that two FlowEndPoints can be connected, the StreamCtrl creates a FlowConnection to bind them and calls connect() on the FlowConnection with the two FlowEndPoints as a parameter. For each pair of matching source/sink FlowEndPoints, the FlowConnection calls set_peer() on the producer FlowEndPoint with the consumer as a parameter. This has the same function as calling set_peer() on the VDev. The FlowConnection then calls go_to_listen() (see Figure 5) on one of the FlowEndPointsk which returns a transport address. It then calls connect_to_peer() on the other FlowEndPointe using the transport address as a parameter. Note that the sink FlowEndPoint is not necessarily the FlowEndPoint on which go_to_listen() is called. The FlowEndPoint on which this is called may be chosen by considerations such as firewall security. For example, if one of the FlowEndPoints is behind a firewall then it must be the listener while the other FlowEndPoint outside the firewall must be the connector.  Tis StreamCtrl implementation’s flow matching algorithm may be designed to be overriden in derived classes so that the algorithm for matching flow endpoints is suited to a particular scenario. Note that the sink FlowEndPoint can be on either on the A side or the B side. The bind call to StreamCtrl will be given the names of the flows to bind and it will first search the A side for a flow of this name to bind and then the B side.

Figure 5 - Flow establishment in full profile Media Streaming Framework

There are two important points to note about flows vs streams in the full profile:

- A stream endpoint can be used to group a collection of flow endpoints which can reside on different nodes. A FlowEndPoint can be added to a StreamEndPoint by calling add_fep() on a StreamEndPoint.

- A FlowConnection can be used exactly like a StreamCtrl except that it only controls one flow. A collection of FlowConnections can be set up and subsequently grouped under a StreamCtrl by calling set_flow_connection() on the StreamCtrl for each FlowConnection. Operations like stop and start applied to the StreamCtrl will be applied to all contained FlowConnections.

## Properties of Streams

The Media Streaming Framework makes extensive use of properties to describe devices, streams, flows and their endpoints. This allows any of these entities to be queried to establish their status and their compatability constraints. Section 2.13 of this document outlines some standard properties of stream entities and their associated values.

## Quality of Service

The application programmer can use either network level or application-level QoS parameters to set up a connection. The application level QoS will be translated to network level QoS internally. The QoS definition is essentially a named list of properties and their values. IDL for the QoS structures is shown below:

// From Property Service

typedef string PropertyName;

struct Property{

PropertyName property_name;

any property_value;

};

typedef sequence<Property> Properties;

struct QoS{

string QoSType;

Properties QoSParams;

};

typedef sequence<QoS> streamQoS;

The QoSType is a convenience label used to group QoS parameters and identify which flow they pertain to. Most operations for modifying or binding streams take a parameter of type streamQoS. This allows the application programmer to specify QoS on a flow-by-flow basis. For example consider a stream which has two flows, one called “video” and one called “audio”. When establishing a stream of this type the application programmer will typically call bind_devs() with a streamQoS parameter which has two elements, one to specify the QoS for the video and one for the audio. For example:

<

{“video”

<

{”video_framerate” 25}

{“video_colourDepth” 8}

>

}

{“audio”

<

{“audio_sampleRate” 8000}

{“audio_numChannels” 2}

>

}

>

There are a couple of things to note about this. Firstly, the QoS for a particular flow is indicated using the name of the flow. Secondly the QoS parameters shown in this example are application level QoS parameters (i.e. they relate to the performance of the application as opposed to the network QoS needed to transport the flow). A number of these QoS parameters are standardised in this document (Section 5.12). It is up to the A/V Streams implementation to translate the application-level QoS parameters such as “video_framerate” to suitable network level QoS parameters such as “Bandwidth”.  If the application programmer prefers he may directly specify the StreamQoS above using Network-level QoS parameters instead of applicaton-level QoS parameters. In this case the above example might become something like the following:

<

{”video”

<

{“ServiceType” 0} // Best effort

{“Bandwidth” 1500000}

{“Bandwidth_Min” 1250000}

{“Delay” 100}

…

>

}

{“audio”

<

{“ServiceType” 1} // Guaranteed

{“Bandwidth” 8000}

{“Delay” 100}

…

>

}

>

A set of common network-level QoS parameters are also specified in this document. There is a reserved QoSType value “Network_QoS”, this is used to indicate a QoS structure which contains only network-level QoS parameters.

Note that there is a convention that where a range of values is acceptable, the desired value should be called “Name” and the acceptable value should be either “Name_Max” or “Name_Min”.

## Stream specification

It is beneficial for a stream service implementation to provide a notation to specify the content of flows and their relative direction within a particular stream type. This notation, however, is beyond the scope of this RFP response. A de facto notation such as TINA-C ODL stream template [9], for example, could be used. It is expected that another RFP will be issued which will cover the area of stream typing notation and the related language mappings for typesafe insertion and extraction of data to and from a flow.

Using a streams notation it should be possible to compile typesafe versions of the interfaces so far discussed. So, for example, a stream of type X will always lead to the generation of a new IDL interface X_StreamCtrl which can only be used to bind X devices. The compiler would also generate the implementation code for the new IDL interfaces. Although the notation itself is not within the scope of this RFP response, the resulting generated IDL interfaces can be standardised.

Consider, for example, a specification for a videophone stream with two flows containing audio and video, one in each direction. Compiling such a stream specification will result in an IDL file which will contain IDL for specialisations of the following interfaces:

- StreamCtrl (videophone_StreamCtrl)

- StreamEndPoint _A and _B (videophone_A, videophone_B)

- VDev (v_videophone)

- MMDevice (videophone)

A Stream notation might typically define a stream as being a composition of different flows. Flows can also be strictly typed as follows, for a flow type X:

- FlowProducer (X_Producer)

- FlowConsumer (X_Consumer)

- FDev (F_X)

- FlowConnection (X_Connection)

The generated IDL for the videophone definitions might look something like the following:

#include <MediaStreams.idl>

interface Videophone_StreamCtrl : StreamCtrl{

…

boolean bind_videophone_devs(in videophone a_party,

in videophone b_party,

inout streamQoS the_qos,

in flowSpec the_spec)

raises(…);

…

};

etc.

The stream notation compiler will generate the source code for implementation classes corresponding to these interfaces (see Figure 6). In Figure 6 the classes labelled ‘user’ are generated as a result of the stream specification. By using a videophone class in place of an MMDevice class and a videophone_StreamCtrl class instead of StreamCtrl, the application programmer can catch gross errors, such as attempting to connect a videophone to a cd_player at compile time. The StreamCtrl, MMDevice and StreamEndPoint classes all contain the property “Type” which allows them to be queried as to the type of stream they support. A value of empty-string indicates that, no typing information is supplied. Each of the interfaces also contains a property “Flows” which contains the names of all the flows in the stream, device or stream endpoint.

It is important to bear in mind that the classes StreamCtrl, StreamEndPoint_A and StreamEndPoint_B are directly instantiable and can be used for general stream control and management. The use of specialised subtypes like videophone_StreamCtrl is merely a convenience for catching errors at compile time and making stream programming simpler.

Figure 6 - Relationships between a subset of objects in the media stream framework

## Flow Protocol

There are a number of ways in which the type of transport used to stream data has a fundamental impact on the solution architecture. To understand why this is the case, it is necessary to realise that there are differences between the transport requirements of a regular ORB and the transport requirements for a multimedia stream. An ORB requires a reliable transport, which implicitly involves retransmission when a frame is dropped or in error. Unfortunately retransmission can seriously impact an isochronous stream by causing jitter. Furthermore, it can add unecessary weight to the transportation of time-critical data if that data is being transmitted on a highly reliable network anyway. It is quite often the case that the timeliness of stream data is more important than whether it is completely correct all of the time. As long as the application is aware that there is a problem, it can deal with it in its own way. With this is mind there are three fundamental types of transport which should be supported by the framework:

- Connection-Oriented Transport: This is provided by transports like TCP and required where completeness and reliability of data are essential.

- Datagram-oriented transport: This is frequently more efficient and lightweight if the application doesn’t mind losing the occasional datagram and handling a degree of mis-sequencing. UDP is used by many popular Internet streaming applications. The framework must insert sequence numbers into the packets to ensure that mis-sequencing and packet-loss is detected.

- Unreliable connection-oriented transport: This is the type of service supplied by ATM AAL5. Messages are delivered in sequence to the endpoint but they can be dropped or contain errors. If there are errors then this fact is reported to the application. There is no flow control unless provided by the underlying network (e.g. ABR).

In addition to the requirements placed on the framework by the various transport types there are application level factors which will influence the way that flows are structured. These include the need for timestamping in some applications. Also there may be a need for adding an indication of the source of a media packet. This is in cases where a stream endpoint may be receiving from multiple senders on an internet multicast connection.  All this information will need to transported ‘in band’. Such information can include (refer to RTP specification [5]):

- Sequence numbers

- Source indicators

- Timestamps

- Synchronisation source

There is no single transport protocol which provides all the capabilities needed for streamed media. ATM AAL5 is good but it lacks flow control so a sender can overrun a receiver. Only RTP provides facilities for transporting the in-band information above, but RTP is internet-centric and it should not be assumed that a platform must support RTP in order to take advantage of streamed media. Furthermore, none of the transports provide a standard way for transporting IDL typed information.

The only way to accommodate the various needs of multimedia transport over a multitude of  transports it is to define a simple specialised protocol which works on top of various transport protocols and provides architecture independent flow content transfer. This protocol will be referred to as the Simple Flow Protocol (SFP). There are two important points to note about SFP:

- It is not a transport protocol, it is a message-level protocol, like GIOP, and is layered on top of the underlying transport. It is simple to implement.

- It is not mandatory for a Media Streaming Framework implementation to support SFP. A flow endpoint which supports SFP can switch it off in order to communicate with a flow endpoint which does not support it. The use of the SFP is negotiated at stream establishment. This is discussed in Section 3.4. If the stream data is not IDL-typed (i.e. It is in agreed byte layout, for example MPEG) then, by default, SFP will not be used. This allows octetstream flows to be transferred straight to a hardware device on the network.

Note that the role of the SFP in this specification is a very important one. It is little use standardising a set of IDL interfaces to manipulate A/V streams if the flows themselves are not interoperable or do not contain the framing and in-band information necessary to provide timely, accurate delivery of multimedia data over a variety of common protocols such as ATM AAL5 and UDP which cannot do this of themselves. These goals are simply unattainable without the specification of SFP. The SFP is not ‘just another protocol’. It is as fundamental to the media streaming framework as GIOP is to the CORBA architecture. Just as CORBA was not complete without GIOP so this standard would be of little value without SFP.

The SFP v1.1 protocol messages are specified as:

module flowProtocol{

enum MsgType{

// Messages in the forward direction

Start,

EndofStream,

SimpleFrame,

SequencedFrame,

Frame,

SpecialFrame,

// Messages in the reverse direction

StartReply,

Credit};

struct frameHeader{

char magic_number[4]; // ‘=’, ‘S’, ‘F’, ‘P’

octet flags;	// bit 0 = byte order,

// 1 = fragments, 2-7 always 0

octet message_type;

unsigned long message_size;	// Size following this header

};

struct fragment{

char magic_number[4]; // ‘F’, ‘R’, ‘A’, ‘G’

octet flags; // bit 1 = more fragments

unsigned long frag_number; // 1,..,n

unsigned long sequence_num;

unsigned long frag_sz;

unsigned long source_id;	// Required for UDP multicast

};						// with multiple sources

struct Start{

char magic_number[4]; // ‘=’, ‘S’, ‘T’, ‘A’

octet major_version;

octet minor_version;

octet flags;  // bit 0 = byte order

};

// Acknowledge successful processing of

// Start

struct StartReply{

char magic_number[4]; // ‘=’,’S’,’T’,’R’

octet flags;  // bit 0 = byte order, 1 = exception

};

// If the message_type in frameHeader is sequencedFrame

// the the frameHeader will be followed by this

// (See also RTP note)

struct sequencedFrame{

unsigned long sequence_num;

};

// If the message_type is Frame then

// the frameHeader is followed by this

// See also RTP note

struct frame{

unsigned long timestamp;

unsigned long synchSource;

sequence<unsigned long> source_ids;

unsigned long sequence_num;

};

struct specialFrame{

frameID context_id;

sequence <octet> context_data;

};

struct credit{

char magic_number[4]; // ‘=’,’C’,’R’,’E’

unsigned long cred_num;

};

};

As in GIOP, the SFP uses CDR encoding and obeys the same conventions. That is, in the frameHeader structure the magic number and flags are in network byte order. All subsequent fields are CDR encoded. All fields in the Start and StartReply messages are also in network byte order

Note that for a simple frame there is no subsequent header, simply a stream of octets. The other message headers: frameHeader, frame and specialFrame are followed by a stream of octets.

Normally, if a frameHeader has the message_type=SequencedFrame then it is followed by a sequencedFrame structure and if it has the message_type=Frame then it is followed by a frame structure. The exception to this rule is when SFP is running over RTP. In this circumstance, the sequencedFrame and frame structures are not used since the values for timestamp, synchSource, source_ids and sequence_num are embedded directly into the corresponding RTP protocol fields [5].

The typical structure of an SFP dialogue for a point-to-point flow is shown in Figure 7. The dialogue begins with a Start message being sent from source to sink. The source waits for a StartReply message. The message_size field in the frame header denotes the size of the message (including the headers) if no fragmentation is taking place. If fragmentation is being used then message_size indicates the size of the first fragment including headers. When fragmentation occurs, the first fragment which begins with the FrameHeader is implicitly fragment number 0. Subsequent framgments associated with this header are labelled fragment number 1 through N.

Figure 7 - Typical SFP dialogue

The sink of a flow may send a credit message to the source of the flow to tell it to send more data. The cred_num field will be incremented with each credit message sent. This facility may be used with protocols such as ATM-AAL5 or UDP which have no flow control. Section 5.10.3 describes the behaviour of SFP in more detail. Note that the SFP dialogue is much simpler on a multicastflow since no messages are sent in the reverse direction.

## Examples for point-to-point streams

The code fragment below illustrates a simple stream binding (see Figure 8):

// C++

// Declare the local and remote videophone multimedia

// devices

videophone_ptr myPhone = …;

videophone_ptr johnsPhone = …;

//Declare the stream controller

videophone_StreamCtrl_ptr myStream;

// Some code here to initialise the local MMDevice (myPhone)

…

// Bind johnsPhone

…

myStream = myPhone->bind(johnsPhone,

someQoS,

&wasQoSMet,	// Was requested QoS honoured

nilFlowSpec); // Bind all flows

myStream->start();

cout << “Hit any key to hang up…” << endl;

cin >> buf;

myStream->stop();

myStream->destroy();

The code fragment below illustrates another way of achieving the same effect.

// C++

// Declare the local and remote videophone multimedia

// devices

videophone_ptr myPhone = …;

videophone_ptr johnsPhone = …;

//Declare the stream controller

videophone_StreamCtrl myStream;

// Some code here to initialise the local MMDevice (myPhone)

…

// Bind johnsPhone

…

// Bind the two devices using a stream with a specified QoS

wasQoSMet = myStream.bind_videophone_devs(

myPhone,johnsPhone,QoSspec,nilFlowSpec);

myStream.start();

cout << “Hit any key to hang up…” << endl;

cin >> buf;

myStream.stop();

myStream.destroy();

One point of interest in the above fragment is that no explicit code for reading from or writing to the stream is shown. This is because each flow has a thread associated with it which loops around reading from the network and writing to the multimedia hardware or vice versa. The application programmer, however, is not compelled to use separately threaded FlowEndPoint. He/she can loop around calling read() or write() style operations on the FlowEndPoint. These operations can be untyped for ‘octetstream’ flows or typed for non-octetstream flows. Another point to note is the use of the specialised, typesafe, bind_videophone_devs() call instead of bind_devs().

The following example shows how StreamEndPoints can be used independently of MMDevices.

One the client side:

// C++

// Details omitted

// Declare local and remote phones

videophone_B_ptr remote_phone;

videophone_A_ptr local_phone = new videophone_A(…);

videophone_StreamCtrl my_stream_controller;

// Bind the remote_phone

…

my_stream_controller.bind(local_phone,

remote_phone,QoSspec,nilFlowSpec);

my_stream_controller.start();

cout << “Hit return to hang up! ” << endl;

cin >> buf;

my_stream_controller.stop();

my_stream_controller->destroy();

Using the StreamEndPoint interface directly, a stream can exist independently of a multimedia device.  The bind family of calls on the StreamCtrl all work in one of two ways:

In the full version of the Media Streaming Framework, the stream compatability rules are used to determine a viable stream setup. For each matching pair of source/sinks, The StreamCtrl calls go_to_listen() on one of the FlowEndPoint pair and connect_to_peer() on the other.

In the light version of the Media Streaming Framework the stream is set up by calling A_end-point->connect(B_EndPoint,QoS,flowSpec) or B_EndPoint->connect(A_EndPoint,QoS,flowSpec). The connect() operation basically works by setting up a number of communications channels to the peer StreamEndPoint (remote_phone). This involves:

- Choosing protocols which are supported by the StreamEndPoint_B

- Performing a QoS translation from application level parameters to protocol specific parameters

- Optionally create ‘sockets’ and start listening on a subset of flows in the StreamEndPoint_A

- Requesting connection of all the flows to StreamEndPoint_B, pass transport addresses of any sockets that are being listened on

- The StreamEndPoint_B sets up ‘sockets’ which listen on the appropriate transport addresses and returns these addresses. Also connects to any listening sockets on StreamEndPoint_A

- The StreamEndPoint_A connects its remianing unconnected flows to addressesof listening ‘sockets’ in StreamEndPoint_B and returns

Figure 8 - Overview of objects involved in a point-to-point videophone stream

## Issues in Modifying QoS

One advantage of having a stream abstraction is that the QoS associated with that stream can be changed over time. One way in which this can happen is through the user directly requesting a quality of service change.

// c++

// Modifying streams

try{

myStream->modifyQoS(newQoS);

}catch(QoSRequestFailed f){

…

}

This is deceptively simple since changing the application level QoS can have complex effects. Any changes to application level QoS must therefore be informed to one or both of the VDevs. When a StreamCtrl receives a modify_QoS() request, it calls modify_QoS() on both of the VDevs in a point-to-point stream or the root VDev in a multipoint stream. Each VDev can assess which the application level changes which need to take place to support the new QoS, these changes will be application specific. If the QoS parameters cannot be met at application level then the QoS may be adjusted to best-effort values. The algorithm to be used by well behaving VDevs is summarised as:

- Check that the QoS modification is valid

- If policy dictates, stop the stream

- Send and re-configuration data needed to the peer device (or to the MCastConfigIf in the multipoint case). The peer device might reject a change in the QoS

- Call modify_QoS() on the StreamEndPoint related to this VDev

- Restart the stream if it has been stopped

When modify_QoS() is called on the StreamEndPoint, it only makes changes to network QoS for flows which originate in that StreamEndPoint.

It should be noted that today’s protocols generally do not support QoS changes. Where a FlowEndPoint does not support QoS modification, the flow may be brought down and restarted with a new QoS by the FlowEndPoint object.

## Issues in Multipoint Streams

Multicasting is a very important network technology since it will form the basis of conferencing and broadcasting applications. At the protocol level, multicasting minimises the bandwidth and complexity required to send information to multiple stations on a network. There are two basic programming models for multicast networking: The internet model and the ATM model. In the internet model, a party who wishes to multicast information finds a free multicast address and starts sending information on that address. Any party who wishes to listen ‘tunes in’ to that multicast address. This is the technique used by the popular MBone applications. If more than one party wishes to broadcast packets to the address then there is nothing preventing this from happening. The underlying transport is UDP and so transmission is unreliable and no flow control is exercised. The second method of multicasting is ATM-style. This involves setting up a Switched Virtual Circuit (SVC) to a destination station. The initiator (A party) can then request the local switch to add parties to that circuit creating a multicast tree with one root (source) and a number of leaves (sinks). The interface to the StreamCtrl hides these differences in the underlying programming model from the application programmer.

This section explains how multicast streams are used by the application. Consider a videoserver stream with two flows, audio and video. The code below shows how an application programmer can add a multipoint source (A party) to such a stream and subesequently add two B parties (sinks). It should be noted that in regular internet multicasting no one party is aware of what other parties are listening to a multicast. This is different in the Media Streaming Framework because each B party is explicitly added to the list of B parties by the StreamCtrl and is given the appropriate multicast addresses to listen on. In the example below, all parties are joined prior to start() being called on the stream. It must be possible, however, to add and remove parties after start() has been called.

// C++

// Multipoint stream binding

//

vidserver_ptr aVidsrc = …;

vidserver_ptr vidsink1, vidsink2;

…

// Declare the stream controller

vidserver_StreamCtrl_ptr myVid = …;

// Bind aVidsrc, vidsink1 and vidsink2

…

// Add the multicast root

myVid->bind_devs(aVidsrc,nilObject,someQoS,nilFlowSpec);

// Add a B party for receiving the a/v data

myVid->bind_devs(nilObject,vidsnk1,someQoS,nilFlowSpec);

// Start the stream

myVid->start();

// Add another B party for receiving the a/v data

myVid->bind_devs(nilObject,vidsnk2,someQoS,nilFlowSpec);

cout << “Hit return to end multipoint” << endl;

cin >> buf;

myVid->stop();

myVid->unbind();

## Extending Stream Management Functionality

The StreamCtrl interface provides no support for performing operations like ‘rewind’ or ‘fast forward’ on a flow. This kind of behaviour depends on the functionality of the device which is the producer of the flow. In order to facilitate this kind of operation, each FlowEndPoint may be associated with 0 or 1 ‘media controllers’. In the light profile the application programmer can access a flow’s media controller through an object property of the VDev called “FlowNameX_related_mediaCtrl”.  In the full profile the media controller can be accessed through the FlowEndPoint using the object property “Related_mediaCtrl”. In both cases an Object reference is returned. This allows the application programmer to narrow the reference to the appropriate type of controller. In Section 2.13 a standard controller interface for stored media is provided.

Returning to the application programmer who wants to just ‘rewind’, he can do this by specifying a media controller for his device as follows:

// IDL

interface rewindable_control{

void rewind();

void stop_rewind();

… etc.

};

He then implements this and associates an instance with the appropriate flow endpoint. He can also extend the StreamCtrl interface to make include rewind functionality (see below). The implementation of rewind() will narrow the Object reference returned by getting a “Related_mediaCtrl” property to a rewindable_control reference.

//IDL

interface myStreamCtrl : StreamCtrl {

void rewind(string flowname);

void stop_rewind(string flowname);

};

## Device and Stream Parameters

Stream establishment and management is subject to a large number of potential parameters for Quality of Service and other attributes. In order to ensure compatibility between different implementations, a standard set of parameters and their values need to be defined. For generic network-level QoS the following parameters are currently registered (see section 5.12):

Network QoS, parameter set 1

- ServiceType - Best Effort, Guaranteed, Predicted

- ErrorFree - True or False

- Delay - long value

- Delay_Max - long value

- Bandwidth - long value

- Bandwidth_Min - long value

- PeakBandwidth - long value

- PeakBandwidth_Min - long value

- TokenRate - long value

- TokenRate_Min - long value

- TokenBucketSize - long value

- TokenBucketSize_Min - long value

- Jitter - float value

- Jitter_Max - float value

- Cost - float value

- Cost_Max - float value

- Protection - short value, 0= default, no encryption, 1= encryption level 1

This parameter set makes no assumptions about the semantics of policing and shaping policies. This is considered to be out of the scope of this submission.

As an alternative to parameter set 1, the following optional Network QoS parameter set is defined:

Network QoS, parameter set 2

- Duplication - enum dup {IGNORE, DELETE}

- Damage - enum dam {DAM_IGNORE, DAM_NOTIFY, DAM_DELETE, DAM_CORRECT}

- Damage_method - Type to be specified

- Reorder - enum reord {REORDER_CORRECT, REORDER_IGNORE}

- Loss - long, {-1 = LOSS_IGNORE, -2 = LOSS_NOTIFY}, positive integer denotes number of retry attempts before the receiver is presumed dead

- Size_Min - long, min bytes in a data unit

- Size_Max - long, max bytes in a data unit

- Size_avg - long, average number of bytes in a data unit

- Size_avg_span - long, number of subsequent data units sent with an interval (ival_const)

- Ival_Const - long, {-2 = IVAL_MAX, -1 = IVAL_ANY}, positive integer denotes constant time interval between transport requests

- Ival_Max - long, the maximum acceptable value if Ival_Const cannot be met

- Delay - long, {0 = DELAY_VOID, -1 = DELAY_ANY, -2 = DELAY_MIN}, DELAY_MIN denotes best effort with minimal delay, DELAY_ANY = best effort, low cost. Positive integer denotes delay required.

- Delay_Max - long, indicates maximum acceptable delay

- Delay_Cum - long, indicates acceptable cumulative delay for concatenated stream

- Jitter - long

- Jitter_Max - long

- ErrDamRatio - float, Ratio of damaged data units {0.0 = RATIO_DAM_VOID, -1.0 = RATIO_DAM_ANY, -2.0 = RATIO_DAM_MIN}, where RATIO_DAM_MIN is a request for minimal error ratio, RATIO_DAM_ANY is request for less costly ratio. A number between 0-1.0 indicates the desired ratio

- ErrDamRatio_Max - float, maximum acceptable error ratio for damaged data units

- ErrLossRatio - float, ratio of lost data units { 0.0 = RATIO_LOSS_VOID, -1.0 = RATIO_LOSS_ANY, -2,0 = RATIO_LOSS_MIN}, where RATIO_LOSS_MIN is a request for minimal error ratio, RATIO_LOSS_ANY is request for less costly ratio. A number between 0-1.0 indicates the desired ratio

- ErrLossRatio_Max - float, maximum acceptable ratio of lost data units

- Workahead_Mode - enum {AHEAD_BLOCKING, AHEAD_NONBLOCKING}

- Workahead_Max - long, the maximum number of data units the producer may be ahead of the consumer

- Playback_Mode - Type to be specified, indicates playback strategy

- Playback_Max - long, The maximum delay introduced by the producer to counter jitter effects

It is mandatory for an implementation of streams to support Network QoS parameter set 1, it is optional to support Network QoS parameter set 2.

Not all of the Network QoS parameters need to be used to describe network level QoS. For example in a toll-free environment where only best-effort limits are used, the “Network QoS” QoS structure could use only the properties: ServiceType, ErrorFree, Delay, Bandwidth and Jitter.

Devices and streams themselves have a number of properties associated with them. These properties can be read by using the Object Property Service interfaces from which StreamCtrl, MMDevice, StreamEndPoint, FlowEndPoint, FlowConnection and VDev are derived. The suggested properties for a StreamCtrl are:

StreamCtrl

- Type - string (empty string implies generic stream type)

- Status - seqflowStatus

- QoS - streamQoS

- Network_QoS - streamQoS

- Flows - sequence of strings, current flows (named from A side)

- A_parties - sequence of StreamEndPoint_A

- B_parties - sequence of StreamEndPoint_B

- flowConnections - sequence of FlowConnection

Some suggested properties for MMDevices are:

MMDevice

- Flows - sequence of flow names supported

- FlowNameX_dir - string, directionality indicators

- FlowNameX_availableFormats - sequence of <format_name> strings

- FlowNameX_SFPStatus - sfp status structure

- FlowNameX_PublicKey - sequence of octets

- MaxStreams - long, maximum of streams supported

- CurrentLoad - float (a percentage) indicates load on physical device

The flowNameX_availableFormats property lists all the possible coders/decoders supported by the device which is associated with that flow. This is designated using a <format_name> striwhich takes the following form:

<format_name> ::= <format_category> [“:” <fname>]

<format_category> ::= “MIME” | “IDL” | “UNS”

The MIME format category is managed by the IETF [7] and is now known there as ‘Media Type’. It is referred to here by its older, more familiar name of “MIME Content-Type”. Valid values for <fname> when the <format_category> is MIME are listed in the registration section 5.12. The <format_category> IDL is used to describe IDL-typed flows. The <fname> will be the full IDL of the flow element. The <format_category> “UNS” indicates unspecified and is not followed by <fname> information. Further <format_category> values can be registered with the OMG (see section 5.12).

The property “FlowNameX_dir” states the possible directions for this flow supported by the device (i.e.  “in”, “out” or “inout”).

The properties for the FDev interface are very similar to those for MMDevice:

FDev

- Flow - string, name of flow supported

- Dir - string, indicates directionality

- AvailableFormats - sequence <format_name> strings

- SFPStatus - SFP status structure

- PublicKey - sequence of octets

- MaxFlows - long, Maximum number of flows supported

- CurrentLoad - float (a percentage) indicates load on physical device

Properties are especially important on the VDev interface. This is because, during the configuration phase, the devices may need to query eachother’s current settings. The properties supported by VDev are:

VDev

- Related_StreamEndPoint

- Related_MMDevice

- Flows - sequence of string

- FlowNameX_dir - string, directionality indicators

- FlowNameX_availableFormats - sequence of <format> strings

- FlowNameX_currFormat - <format> string

- FlowNameX_devParams - Properties

- FlowNameX_SFPStatus - sfp status structure (only where SFP is in use)

- FlowNameX_status - flow status structures

- FlowNameX_related_mediaCtrl - Object

- FlowNameX_PublicKey - sequence of octets

The flowNameX_availableFormats property states which formats this VDev can support (e.g. MPEG, MJPEG, etc.). The flowNameX_currFormat property states which of these is currently in use. The flowNameX_related_mediaCtrl property holds a reference to a media controller for a flow. These media controllers can be used to implement functionality like rewind and fast forward and can support any interface at all. Section 3.7 supplies a useful standard media control interface whereas Appendix C illustrates how DAVIC’s DSM-CC UU commands could be used to perform media control.

The flowNameX_devParams property states the settings associated with the current codec or device. This document describes the following common device parameters for audio, video and other devices (see also section 5.12):

“language” - string, from the set {…,“English(UK)”,”English(US)”,…,”Irish”,…}

"audio_sampleSize" - short, number of bits per sample

"audio_sampleRate" - long, Hertz

"audio_numChannels" - short

"audio_quantisation" - short, 0 = linear, 1 = u-law, 2 = A-law, 3 = GSM

"video_framerate" - long

"video_colourDepth" - short, e.g. 2, 4, 8, 12, 16, 24, 32

"video_colourModel" - short 0 = RGB, 1 = CMY, 2 = HSV, 3 = YIQ, 4 = HLS

"video_resolution" - struct resolution.

Further properties can be exposed through a registration process described later in this document.

The properties exposed by a StreamEndPoint are:

StreamEndPoint

- Related_VDev

- Related_StreamCtrl

- Negotiator - A negotiator object ref.

- Flows - sequence of flow names supported

- FlowNameX_dir - string, directionality indicators

- FlowNameX_currFormat - string, <format> string

- FlowNameX_address - string indicates protocol and address

- FlowNameX_status - stopped, started, destroyed

- FlowNameX_flowProtocol - string (<flowProtocol>)

- FlowNameX_PublicKey - sequence octet

- AvailableProtocols - sequence of string (protocol names)

- ProtocolRestriction - sequence of string (protocol names)

- PeerAdapter - StreamEndPoint reference

The AvailableProtocols property states what protocols are available to this StreamEndPoint. The ProtocolRestriction property lists the restriction currently placed on what protocols may be used for the purposes of connecting to another StreamEndPoint.

The flowNameX_address property is formatted according to <transport_address> syntax described later in this document. For example, it could typically have the format “TCP=cod.fish.net:2222”.  The directionality is expressed from the A-side, so if the direction of a flow is “in” on a B end-point that means that the flow is originating on the B side and terminating on the A side.

The properties exposed by a flow endpoint are:

FlowEndPoint

- FlowName - string

- Format - sequence of <format> string

- CurrFormat - <format> string

- DevParams - property list, describes

- Status - Stopped/started

- FlowProtocol - string (<flow protocol>)

- Active - Boolean

- Dir - enum sink/source (State actual type name)

- flowProtocol - string, flow protocol name in <flowProtocol> syntax e.g. “SFP1.0”

- SFPStatus - SFP status structure

- Related_mediaCtrl - Object, the related media controller

- Address - Formatted string

- AvailableProtocols - protocol spec

- CurrProtocol - <protocolname>

# IDL Interfaces

The purpose of this section is to specify the semantics of each IDL interface in more detail..

## The Basic_StreamCtrl

The first and most important interface is the StreamCtrl interface. The StreamCtrl interface inherits from Basic_StreamCtrl.

struct SFPStatus{

boolean isFormatted;

boolean isSpecialFormat;

boolean seqNums;

boolean timestamps;

boolean sourceIndicators;

};

interface Basic_StreamCtrl : PropertyService::PropertySet {

// Empty flowSpec => apply operation to all flows

void stop(in flowSpec the_spec) raises (noSuchFlow);

void start(in flowSpec the_spec) raises (noSuchFlow);

void destroy(in flowSpec the_spec) raises (noSuchFlow);

boolean modify_QoS(inout streamQoS new_qos,

in flowSpec the_spec)

raises (noSuchFlow, QoSRequestFailed);

// Called by StreamEndPoint when something goes wrong

// with a flow

oneway void push_event(

in streamEvent the_event);

void set_FPStatus(in flowSpec the_spec,

in string fp_name,		// Only SFP1.0 currently

// specified

in any fp_settings)		// Currently SFP accepts

// SFPStatus structure

raises (noSuchFlow, FPError);

Object get_flow_connection(in string flow_name)

raises (noSuchFlow, notSupported);

void set_flow_connection(in string flow_name,

in Object flow_connection)

raises (noSuchFlow, notSupported);

};

The property service is used to record the properties enumerated in the previous section. The properties are readonly.

When a point-to-point stream receives a start() operation it does two things. It first determines whether there is a related Media Control object for that flow (i.e. it queries the FlowNameX_related_mediaCtrl property on the VDev in the light profile or queries the Related_mediaCtrl property on the relevant FlowProducer in the full profile. It then calls start() on the media control object with a relative position of 0.  It then calls start() on the A-party stream endpoint. The A-party end-point could do the following:

- Calls start() on its incoming flows.

- Calls start() on its peer B-party end-point.

- Calls start() on its outgoing flows.

The start() operation takes a sequence of flow names (called a flowSpec) as a parameter. If the list is empty then the operation is applied to all flows. This convention is used throughout these interfaces. The stop() operation works in exactly the opposite way to the start() operation. In multipoint streams, only the A-party is started and stopped, the B-party is always ready to read.

The destroy() operation tears down a stream. This includes tearing down the transport connections and deleting all MMDevice-created StreamEndPoints and VDevs.

The modify_QoS() operation changes the QoS associated with a stream. The operation of the modify_QoS() operation has already been discussed in section 2.10. If the resulting QoS was ‘best effort’ then it returns false and the QoS parameter’s out value will indicate the actual modification to the QoS.

The push_event() operation is used by StreamEndPoints or VDevs to inform the StreamCtrl of events that are happening on a flow. Events are just typed as properties. The following events are currently defined:

- {“FlowLost” string} where string is the name of the flow lost and

- {“FlowReEstablished” string} where string is the name of the flow re-established.

- {“QoSChanged” string} where the string holds the name of the flow.

The set_FPStatus() operation allows the application programmer to explicitly control aspects of flow protocol for all flows in the stream. It is envisaged that setting the flow protocol parameters of a stream will seldom be done explicitly at this level but the hooks to allow it are there. There will be further discussion of this feature under the StreamEndPoint interface.

The set_flow_connection() operation will raise a notSupported exception in “light” profile implementations. In full profile implementations it will allow the declaration and installation of a particular FlowConnection object for flow. This is desirable where an application programmer may want a FlowConnection to have special, inherited functionality. For example:

interface specialFlowConnection : FlowConnection, MediaControl {};

The application programmer can declare an instance of specialFlowConnection to handle a particular flow, say a video flow, and then call

myStreamCtrl->set_flow_connection(“video1”,myspecialFlowConnection);

## The StreamCtrl

The StreamCtrl interface for streams is specified below. Its purpose is to allow for point-to-point and point-to-multipoint bindings. The StreamCtrl interface was kept separate from Basic_StreamCtrl to allow the future possibility of other binding controllers which inherit from Basic_StreamCtrl. The StreamCtrl is described as follows:

interface StreamCtrl : Basic_StreamCtrl {

boolean bind_devs(in MMDevice a_party, in MMDevice b_party,

inout streamQoS the_qos,

in flowSpec the_flows)

raises (streamOpFailed, noSuchFlow, QoSRequestFailed);

boolean bind(in StreamEndPoint_A a_party,

in StreamEndPoint_B b_party,

inout streamQoS the_qos,

in flowSpec the_flows)

raises (streamOpFailed, noSuchFlow, QoSRequestFailed);

void unbind_dev(in MMDevice dev,

in flowSpec the_spec)

raises (streamOpFailed, noSuchFlow);

void unbind_party(in StreamEndPoint the_ep,

in flowSpec the_spec)

raises (streamOpFailed, noSuchFlow);

void unbind()

raises (streamOpFailed);

VDev get_related_vdev(in MMDevice adev,

out StreamEndPoint sep)

raises (streamOpFailed);

};

interface MCastConfigIf : PropertyService::PropertySet{

boolean set_peer(in Object peer,

inout streamQoS the_qos,

in flowSpec the_spec)

raises (QoSRequestFailed, streamOpFailed);

void configure(in PropertyService::Property a_configuration);

void set_initial_configuration(

in PropertyService::Properties initial);

// Uses <format_name> standardised by OMG and IETF

void set_format(in string flowName, in string format_name)

raises (notSupported);

// Note, some of these device params are standardised by OMG

void set_dev_params(in string flowName,

in PropertyService::Properties new_params)

raises(PropertyService::PropertyException,

streamOpFailed);

};

The basic bind_devs() operation sets up a stream between MMDevices: streamQoS identifies the desired QoS for the stream. If it is not met then the bind_devs() call returns false and the streamQoS parameter is changed to reflect the actual QoS of the stream.

The flowSpec is used to name the subset of flows to bind. If the flowSpec is an empty sequence then all flows are bound. If it was not possible to bind all of the requested flows, the StreamCtrl has two options, it may throw a streamOpFailed exception or it may return false. In the latter case the streamQoS parameter will reflect the flows which the StreamCtrl succeeded in binding and the QoS for those flows. The name of each bound flow can be read from the QoSType field of each QoS structure in the streamQoS list.

The bind_devs() call can be used in the following modes:

bind_devs(aDev,bDev,someQoS,flowSpec)

Bind two MMDevices, aDev and bDev with a stream

bind_devs(aDev,nilObject,someQoS,<f1,f2>)

Bind aDev as a multipoint source with the flows f1 and f2.

If aDev is already bound then the effect is to add the flows f1 and f2 to the stream.

For example, the A party will now be multicasting f1 and f2 as well as whatever flows it was previously multicasting. Any subsequent B parties which join will a nilflowSpec will automatically have access to these flows but existing B parties will not.

bind_devs(nilObject,bDev,-,<f1,f2>)

Bind bDev as a multipoint sink with the flows in flowSpec

If bDev is already bound then add flows from f1 and f2, ie. the bDev will now ‘listen’ for multicast flows f1 and f2.

bind_devs(nilObject,nilObject,someQoS,<f1,f2>)

Add the flows f1 and f2 to all parties in an existing stream. In the multipoint case this means all existing A parties and B parties will multicast/receive the flows f1 and f2.

In multipoint streams between VDevs, configuration information is distributed via the MCastConfigIf interface, shown above. The configfuration commands (e.g. set_format(), configure()) are sent by A party devices to an MCastConfigIf. The semantics of the configuration operations on the MCastConfigIf is the same as their equivalents on the VDev interface except that all configuration calls will be distributed by the MCastConfigIf to B parties connected to the stream. How this information is distributed to the B parties is not stipulated in this submission. It is possible (if multicast messaging is not available) for the MCastConfigIf object to make configuration calls on each of the B parties in turn.

Calls to configure() use parameters of type Property, for example:

{

“video_interlace”

True

}

When a B party joins it should receive some initial configuration information to tell it how the A party device is configured. The A party can set this initial configuration information through the set_initial_configuration() operation on MCastConfigIf.

The bind() operation is the equivalent of the bind_devs() operation for StreamEndPoints. Its modes of operation work in just the same way.

The unbind() operation is used to unbind parties from a stream or unbind flows. A sample of the different modes used are as follows:

unbind(aSEP,nilFlowSpec)

Unbind the StreamEndPoint aSEP

unbind(aSEP,<f1,f2>)

Unbind only the flows f1 and f2 from sSEP

unbind(nilObject,<f1,f2>)

Unbind flows f1 and f2 from all stream endpoints

unbind(nilObject,nilflowSpec)

Unbind the stream. Equivalent to calling unbind() which is equivalent to calling destroy().

The get_related_vdev() call allows the application programmer to retrieve information about the endpoints of a stream which has been established. When the application programmer calls bind_devs() on an MMDevice this leads to either create_A() or create_B() being called on the MMDevice(s) passed into this call. When the call to create_A() or create_B() is finished it returns references to StreamEndPoint and VDev objects unique to this StreamCtrl. These references are not given back to the application programmer but are stored internally by the StreamCtrl. The application programmer may access the StreamEndPoint or VDev created by a particular MMDevice for this StreamCtrl, by calling get_related_vdev() on the StreamCtrl passing in a reference to the MMDevice in question. If the MMDevice is unknown or the StreamCtrl cannot comply for some reason, a streamOpFailed exception is thrown.

A stream notation compiler will generate specialised StreamCtrl interfaces which inherit from StreamCtrl. Consider the example definition previously given for the videophone stream (section 2). This would lead to the following code being generated for a specialised StreamCtrl:

Videophone_StreamCtrl : StreamCtrl{

boolean bind_videophone_devs(in videophone a_party,

in videophone b_party,

inout streamQoS the_qos,

in flowSpec the_flows)

raises (streamOpFailed, noSuchFlow, QoSRequestFailed);

boolean bind_videophone(in videophone_A a_party,

in videophone_B b_party,

inout streamQoS the_qos,

in flowSpec the_flows)

raises (streamOpFailed, noSuchFlow, QoSRequestFailed);

boolean bind_videophone_A_party(in videophone_A a_party,

inout streamQoS the_qos,

in flowSpec the_flows)

raises (streamOpFailed, noSuchFlow, QoSRequestFailed);

boolean bind_videophone_B_party(in videophone_B b_party,

inout streamQoS the_qos,

in flowSpec the_flows)

raises (streamOpFailed, noSuchFlow, QoSRequestFailed);

};

## The StreamEndpoint

The StreamEndPoint is described in IDL as follows:

interface StreamEndPoint : PropertyService::PropertySet{

void stop(in flowSpec the_spec) raises (noSuchFlow);

void start(in flowSpec the_spec) raises (noSuchFlow);

void destroy(in flowSpec the_spec) raises (noSuchFlow);

boolean connect(in StreamEndPoint responder,

inout streamQoS qos_spec,

in flowSpec the_spec)

raises (noSuchFlow, QoSRequestFailed, streamOpFailed);

boolean request_connection(

in StreamEndPoint initiator,

in boolean is_mcast,

inout streamQoS qos,

inout flowSpec the_spec)

raises (streamOpDenied, noSuchFlow,

QoSRequestFailed, FPError);

boolean modify_QoS(inout streamQoS new_qos,

in flowSpec the_flows)

raises (noSuchFlow, QoSRequestFailed);

boolean set_protocol_restriction(in protocolSpec the_pspec);

void disconnect(in flowSpec the_spec)

raises (noSuchFlow, streamOpFailed);

void set_FPStatus(in flowSpec the_spec,

in string fp_name,

in any fp_settings)

raises (noSuchFlow, FPError);

Object get_fep(in string flow_name)

raises (notSupported, noSuchFlow);

string add_fep(in Object the_fep)

// Can fail for reasons {duplicateFepName, duplicateRef}

raises (notSupported, streamOpFailed);

void remove_fep(in string fep_name)

raises (notSupported, streamOpFailed);

void set_negotiator(in Negotiator new_negotiator);

void set_key(in string flow_name, in key the_key);

void set_source_id(in long source_id);

};

A StreamEndPoint may be associated with a VDev, if this is the case then the Related_VDev property holds a reference to the VDev.

The start() and stop() and destroy() operations have already been discussed in relation to streams.  Calling destroy() will have the effect of disconnecting all the flow transports rather than destroying the StreamEndPoint objects.

### connect() and request_connection()

Figure 9 - Message Sequence Chart for a connect() operation

The StreamEndPoint interface provides the connect() operation for connecting to a peer StreamEndPoint object. This operation is normally used in the ‘light’ profile of the stream component and is usually called from an A party to a B party. The ‘full’ profile, where available, will perform stream setup using FlowEndPoint interfaces. The workings of connect() are illustrated by the message sequence chart in Figure 9.  The sequence in which an A end-point makes calls to the B end-point is very important since, in order for different implementations of StreamEndPoint from different vendors to interwork, this sequence of operations must be followed.

// Pseudo-code

boolean StreamEndPoint_A::connect(

in StreamEndPoint_B_ptr remoteStreamEp,

inout streamQoS aQoS,…)

If there is no local negotiator, then

there will be no negotiation

else

tmpAny = remoteStreamEp

->get_property_value(“Negotiator”);

// extract peerNegotiator from tmpAny

…

myNegotiator->negotiate(peerNegotiator,

aQoS);

Choose protocols based on what the remote end-point can support. If the QoS is specified at application level then translate application-level QoS into network level QoS for those protocols.

Set up listening flows where appropriate

try{

remoteStreamEp->request_connection(

this,

false		// Not multipoint

offeredQoSspec, // inout

flow_spec	// inout

);

}catch(…){

Take appropriate action

}

Connect to listening flows in the B-party where appropriate

using transport addresses from the flow spec

}

#### Flow Specification

An important point for interoperability is the format of the flowSpec. This is a crucial piece of data since it conveys the information needed to set up the transport connections which carry the flows. The flowSpec data type is a sequence of strings but each string must be formatted according to strict rules. Before describing the structure of these strings it is important to establish a basic rule: The StreamEndPoint which supports the flow consumer endpoint is not necessarily the StreamEndPoint which listens for the transport connection for that flow. The specification is written in such a way that either the producer end or the cosumer end can decide to listen for the transport connection which carries the flow. The structure of the flowSpec will indicate to the peer StreamEndPoint whether the ‘A-party’ is already listening for a flow or whether it expects its ‘B-party’ peer to listen for the flow.

For the critical request_connection() call, a flowSpec is either a forward flowSpec or a reverse flowSpec. A forward flowSpec is going from an A party to a B party (i.e. the the_spec parameter going in) and the reverse flow spec is going back from B to A (i.e. the the_spec parameter’s out value). The syntax is expressed below:

<flowSpec> ::= <flowName> [“\” <forwardFlowSpec> | <reverseFlowSpec>]

<forwardFlowSpec> ::= <direction> [“\” [<format_name>] [“\” <flowProtocol> “\”

[<address>] [“\” <address> ]]]

<reverseFlowSpec> ::= [<address> [“\” <flowProtocol> [“\”<direction> [“\” <format_name>]]]]

<direction> ::= “in” | “out”

<flowProtocol> ::= <flowProtocolName> “:” <version> [“:” <flowProtocolOptions>]

<flowProtocolName> ::= “SFP” | …

<flowProtocolOptions> ::= <OptionTag> [“=” <OptionValue>] [“;” <flowProtocolOptions>]

<OptionTag> ::= <AlphaNumericString>

<OptionValue> ::= <AlphaNumericString>

<address> ::= <carrier_protocol> [“=” <networkAddress> “;” <portList>]

<carrierProtocol> ::= “TCP” | “UDP” | “AAL5” | “AAL3-4” | “AAL1” | “RTP/UDP” | “RTP/AAL5” | “IPX” | …

<portList> ::= <portNumber> [“;” <portList>]

<portNumber> ::= <intstring> //  > 0

<version> ::= <major_version> “.” <minor_version>

<major_version> ::= <intstring>

<minor_version> ::= <intstring>

<format_name> ::= <format_category> [“:” <fname>]

<format_category> ::= “MIME” | “IDL” | “UNS”

<intstring> ::= stringified integer

The ellipsis (…) following the productions <flowProtocolName> and <carrierProtocol> indicate that further values for these can be registered with the OMG (see section 5.12). The currently recognised values for <OptionTag> are “Credit”. The corresponding <OptionValue> will denote a positive, non-zero integer e.g. Credit=10.

Example 1:

An example of a simple forward flowSpec might be: < "video1\out\MIME:video/MPEG" >. This flowSpec tells the B party that the A party wants to establish one flow called video1. The flow is ‘out’-bound so it flows from the B party to the A party. The format of the flow is video/MPEG. The B end-point will process this and might typically output something like the following: <“video1\TCP=cod.fish.net;5678”>. This confirms that the flow video1 has been established and a TCP socket is waiting on transport address cod.fish.net;5678.  Note that the sink of the connection does not have to be the party which goes into listen mode for the connection.

It is important to note at this point that when streams are being established between administrative domains (e.g. from between a consumer and a telephone company), it will not be possible to co-ordinate the naming of the flows. With reference to this example, although there is a flow named “video1” in StreamEndPoint_A, there may not be in StreamEndPoint_B. In these cases the StreamEndPoint_B can use information about the directionality “out” and format “MIME:/video/MPEG” of the flow from the flowSpec to dynamically match the flow name “video1” to its equivalent in StreamEndPoint_B, say “myHomeVideo”. The StreamEndPoint_B then stores infomation to the effect that “video1” maps to “myHomeVideo” for the duration of the current connection. If it receives a request to stop() or start() “video1” it will interpret this as a request to stop or start “myHomeVideo”.

Example 2

In this example the forward flowSpec is < “audio1\out\\SFP:1.0:Credit=10” >.

That is, the flow audio1 is to be established with the A party as the sink. No MIME format is given. The SFP v1.0 is to be used and the sink wishes to have 10 frames of credit for flow control. This implies that a protocol such as ATM AAL5, which does not use flow control, has been chosen. The credit mechanism gives the sink of the flow a simple way to stop itself being overrun by the source. After every 10 frames (or whenever a timeout occurs), it must send a credit message back to the source using the SFP source, in turn, stops after every ten frames it has sent and waits for the receiver to acknowledge that it has caught up. The request_connection call may fail with exceptions that indicate that the B party does not support the formatting protocol or that it cannot use flow control. If successful the reverse flowSpec will look something like the following: < “audio1\AAL5=…” >.

Example 3

In this example the forward flowSpec is < “audio1\out\\SFP:1.0:Credit=10\AAL5=…” >. This is essentially the same as the above except that the A party is offering to listen for connection establishment. The reverse flowSpec will simply be < “audio1” >.

Example 4

In this example the forward flowSpec is: <simdata\out\UNS:user/simdata\SFP:1.0\\UDP=analogue;5432” >. This time, the A party is the source of the flow. It has been decided already by the A party that the flow will be carried on UDP. The format name is user defined (i.e. unspecified) and indicates to the application that information of type user/simdata is being carried by the flow. The SFP:1.0 field indicates that the flow should use flow protocol SFPv1.0. No credit is sought because the A party is the source of the flow and only sinks can ask for credit. The address given in this context is the address of a reverse channel. A reverse channel is only needed when a datagram-oriented transport like UDP is in use. This reverse channel has two purposes:

- For credit messages to be sent from the sink to the source, the sink will need an address to send the credit datagrams to.

- It is used at the sink to filter out packets which are not coming from the flow source. Because UDP is not connection oriented, datagrams from other sources could potentially corrupt the flow unless this is done.

Typically the return flowSpec might contain a request for credit e.g. < “simdata\UDP=digital;5417\SFP:1.0:Credit=20” >. If, on the other hand, the reverse flowSpec is < “simdata\UDP=digital;5417” >, then no credit is being sought.

There is one exceptional circumstance which causes behaviour to differ from the above.  If the A party above had left the choice of protocol to the B party i.e. the forward flowSpec is: < “simdata\out\UNS:user/simdata\SFP:1.0> and the B party chose UDP. In this case the A party will need to supply the B party with a reverse channel address using another call to request_connection() with the forward flowSpec set to < “simdata\out\UNS:user/simdata\SFP:1.0\\UDP=analogue;5432” >. The B-party will not commence listening for packets from the A party until it has received this reverse channel address.

Example 5

In this example, the A/V Streams specification is being used in an ‘Internet Browser’ type scenario. The StreamCtrl and StreamEndPoint_A are in the browser. StreamEndPoint_B is on the server, behind a firewall. The user finds out about three flows on the server “VideoIn”, “VideoOut” and “TextOut” to which he wishes to connect. The format of these flows is, as yet, unknown. connect() is called on StreamEndPoint_A with these three flows named in the flowSpec. StreamEndPoint_A is set to a mode where it does not attempt to listen on any flows or match the flow names with any of its own flows it simply calls request_connection(). The forward flowSpec for the flow “VideoOut” will simply look like this:  <VideoOut>. When this arrives at the StreamEndPoint_B, it will find the matching flows and send back a reverse flowSpec. The element for flow “VideoOut” will look something like this: <VideoOut\UDP=…\\out\MIME:video/MPEG>. The StreamEndPoint_A will check whether it has a flow endpoint which can match “VideoOut” based on format and direction. Suppose it does find such a FlowEndPoint, “myVideoFlow”, it dynamically maps the “VideoOut” flow to “myVideoFlow” and connects the flow.

### Other Functions Supported by StreamEndPoint

The modify_QoS()operation on this interface refers to modification of transport QoS rather than application QoS. If the transport supporting a flow does not support QoS modification then it may tear down the flow and restart it.

The operations get_fep(), add_fep() and remove_fep() are all implemented in the full profile specification but are not implemented in the light profile, where they raise notSupported exceptions with the value “Full profile not supported”. The get_fep() operation returns a named flowEndPoint object. The add_fep() operation adds a named FlowEndPoint to the StreamEndPoint. The returned string indicates the name of the flow endpoint that was added. This is discovered through the StreamEndPoint querying the FlowEndPoint’s “FlowName” property. If this property does not exist then a name will be generated by the StreamEndPoint.

The operation set_negotiator() may be used by a stream endpoint to attach a negotiating object to the stream endpoint. Negotiator objects are used as a ‘catch-all’ to implement special, user defined behaviour during stream setup. This negotiator object must be derived from the interface Negotiator, (which is shown in the Appendix) but will be highly specialised to a particular type of negotiation.

The set_protocol_restriction() operation is used to restrict the set of protocols which may be used by a stream endpoint when creating a stream. This knowledge can be used, for example, to stop the A party end-point selecting a set of protocols which aren’t supported by the B party end-point. Suppose that the A end supports TCP and AAL5 and the B end supports AAL5 and UDP. By setting the restriction set in A to {“AAL5”, “UDP”} the AAL5 will be chosen for connection set up since it is the only protocol in common with the A sides available set {“TCP”, “AAL5”}. Using an empty list clears the restriction set. Strictly speaking every call to bind_devs() or bind() (in point-to-point cases) should include the behaviour that the StreamCtrl queries one of the StreamEndPoints for its “AvailableProtocols” property and then calls set_protocol_restriction() on the other StreamEndPoint before calling connect() on that StreamEndPoint. In practice however most application programmers will have their system configured on a single network so the protocol restriction checking will be turned off.

The set_source_id() gives the end-point a unique number which can be tagged onto in-band SFP data to identify where it has come from. This can be important in applications which use multicast UDP. For example, consider the case of a flow listening on a UDP multicast address. There are two A-parties sending data to this multicast address. The flow is receiving a sequence of frames but the only way it knows which A party sent which frame is by examining the source-id of the frame. The StreamCtrl ensures that each new A or B party added to a stream receives a unique source id to allow its frames to be uniquely identified on such a multicast address. A VDev will be given this unique source id by the StreamCtrl which can then set its stream endpoints to carry this identifier if needs be.

The set_FPStatus() operation results in flags being set for the flow protocol used by all flows. There are various types of format information which can be transmitted in band using SFP, these are: sequence numbers, timestamps, and source indicators. The sequence numbers are always sent for a UDP-based flow. The sourceIndicators are taken from the source_id() (see previous operation). This information can be conveyed by specifying fp_name as “SFP1.0” and fp_settings as a struct SFPStatus.

The set_key()operation can serve two functions. If public key encryption is being used then this operation can be called by the StreamCtrl to set a shared secret key for the encryption of a flow in the stream. If public key encryption is being used then this call is made by the VDev and informs the local StreamEndPoint of the public key of the peer StreamEndPoint (it is assumed that the local private key is known to the StreamEndPoint, this is considered an implementation detail). The public keys can be exchanged by VDevs during the configuration phase when set_peer() has been called. The VDev supports a standardised property, FlowNameX_PublicKey.

## The StreamEndPoint_A and StreamEndPoint_B

The StreamEndPoint_A and StreamEndPoint_B classes are derived from StreamEndPoint. The StreamEndPoint_A interface is shown below:

interface StreamEndPoint_A : StreamEndPoint{

boolean multiconnect(inout streamQoS the_qos,

inout flowSpec the_spec)

raises (noSuchFlow, QoSRequestFailed, streamOpFailed);

boolean connect_leaf(in StreamEndPoint_B the_ep,

inout streamQoS the_qos,

in flowSpec the_flows)

raises (streamOpFailed, noSuchFlow,

QoSRequestFailed, notSupported);

void disconnect_leaf(in StreamEndPoint_B the_ep,

in flowSpec theSpec)

raises(streamOpFailed, noSuchFlow);

};

interface StreamEndPoint_B : StreamEndPoint {

boolean multiconnect(inout streamQoS the_qos,

inout flowSpec the_spec)

raises (streamOpFailed, noSuchFlow,

QoSRequestFailed, FPError);

};

### Multipoint Operations

The StreamEndPoint A and B interfaces support multipoint operations  such as multiconnect(), connect_leaf(). The multiconnect() operation is used with Internet-style multicast and connect_leaf() is used with ATM-style multicast. The sequence of events when an application programmer sets up a multipoint connection is as follows:

- The application programmer calls StreamCtrl->bind_devs(Adev,nil,…). The StreamCtrl calls A_SEP = Adev->create_A(…,out AVDev,…). It then creates mc, an instance of MCastConfigIf and calls AVDev->setMCastPeer(mc,…).

- When the application programmer calls StreamCtrl->bind_devs(nil,Bdev,…), the StreamCtrl calls B_SEP = Bdev->create_B(…,out BVDev,…). It then calls mc->setPeer(BVDev,…) followed by A_SEP->connect_leaf(B_SEP,…). If this completes sucessfully then B_SEP is bound to a multipoint tree with A_SEP being the root. If connect_leaf() returns an exception of type notSupported with reason “noMCastTreeSupported” then it retries but using multiconnect() instead. It is permitted for the StreamCtrl to test whether a StreamEndPoint_A supports ATM-style multipoint by first calling connect_leaf() with the ‘the_ep’ set to a nil object reference. The multiconnect() call returns a flowSpec with internet multicasting addresses for each of the flows. StreamCtrl calls multiconnect() on B_SEP using this flowSpec.

The StreamEndPoint::disconnect() and disconnect_leaf() operations are used to respectively tear down connection (internet multicast A party or any B party)  and to remove a leaf from a multicast tree.

### Specialisations of StreamEndPoint A and B

The stream notation compiler will generate specialised interfaces for the endpoints of streams. For the videophone example, these would be:

interface Videophone_A : StreamEndPoint_A {

};

Similarly for videophone_B:

interface Videophone_B : StreamEndPoint_B {

};

## The MMDevice

The interface for MMDevice is:

interface MMDevice : PropertyService::PropertySet {

StreamEndPoint_A create_A(

in StreamCtrl the_requester,

out VDev the_vdev,

inout streamQoS the_qos,

out boolean met_qos,

inout string named_vdev,

in flowSpec the_spec)

raises(streamOpFailed, streamOpDenied, notSupported,

QoSRequestFailed, noSuchFlow);

StreamEndPoint_B create_B(

in StreamCtrl the_requester,

out VDev the_vdev,

inout streamQoS the_qos,

out boolean met_qos,

inout string named_vdev,

in flowSpec the_spec)

raises(streamOpFailed, streamOpDenied, notSupported,

QoSRequestFailed, noSuchFlow);

StreamCtrl bind(in MMDevice peer_device,

inout streamQoS the_qos,

out boolean is_met,

in flowSpec the_spec)

raises (streamOpFailed, noSuchFlow, QoSRequestFailed);

StreamCtrl bind_mcast(in MMDevice first_peer,

inout streamQoS the_qos,

out boolean is_met,

in flowSpec the_spec)

raises (streamOpFailed, noSuchFlow, QoSRequestFailed);

void destroy(in StreamEndPoint the_ep, in string vdev_name)

// ie VDev not found

raises (notSupported);

string add_fdev(in Object the_fdev)

raises(notSupported, streamOpFailed);

Object get_fdev(in string flow_name)

raises(notSupported, noSuchFlow);

void remove_fdev(in string flow_name)

raises (notSupported, noSuchFlow);

};

The create_A() and create_B() operations return references to (A and B) StreamEndPoint objects each of which have an associated VDev object passed back in the out parameter the_vdev. The namedDev parameter allows the caller to specify a particular subsystem in the logical device (for example “camera1”). This is an inout value so it can also be used by the MMDevice to tell the streamCtrl what the logical name of the endpoint is. This can later be used to expedite the process of destroying an endpoint. The met_qos parameter indicates whether the QoS of the returned virtual device meets the criteria specified in the ingoing QoS parameter. If the QoS was not met then the actual QoS will be given on the out value of the_qos parameter. The exception streamOpFailed can be raised as a result of badly formed parameters or a system failure. The streamOpDenied exception can be used to indicate that the device is overloaded and cannot take any more connections. If this is the case, then reason “Device overload” should be used. The notSupported exception is thrown to indicate that, if an A endpoint was requested only B endpoints are supported and vice versa. The reason will be given as  “Device supports only A endpoints” or “Device supports only B endpoints”.

The bind() operation creates a stream for binding one MMDevice to another. Calling bind_mcast() creates a multipoint binding and adds the first MMDevice sink to the multipoint stream.

The destroy() operation will remove a StreamEndPoint and its associated VDev. The destroy() operation can either use a StreamEndPoint reference or the string that was returned by the named_vdev parameter in the create_A or create_B operations.

In full profile implmentations, an MMDevice can act as a container for a number of FDevs. In such cases create_A() and create_B() will normally result in calls to the contained FDevs which will return FlowEndPoints. A StreamEndPoint will be created and the FlowEndPoints will be added to it by calls to add_fep(). Generally, no VDev will be created because the FlowEndPoints individually contain functions like set_peer(). In order to fill the MMDevice container with FDevs calls can be made to add_fdev(). Each FDev has a name (in its properties) this is what is returned in a string. If the FDev has no name then one is assigned by the MMDevice. This is analogous to add_fep() on StreamEndPoint. The get_fdev() operation returns a named FDev, while remove_fdev() removes an FDev from the MMDevice. All of these operations raise the exception notSupported with reason “Full profile not supported” in light profile implementations.

The stream notation compiler will generate the following specialised interface for the videophone example:

interface Videophone : MMDevice{

Videophone_A create_videophone_A(

in StreamCtrl the_ctrl,

out v_Videophone the_vdev,

inout streamQoS the_qos,

out boolean met_qos,

inout string named_vdev,

in flowSpec the_spec)

raises(streamOpDenied, streamOpFailed, notSupported,

QoSRequestFailed, noSuchFlow);

Videophone_B create_videophone_B(

in StreamCtrl the_ctrl,

out v_Videophone the_vdev,

inout streamQoS the_qos,

out boolean met_qos,

inout string named_vdev,

in flowSpec the_spec)

raises(streamOpDenied, streamOpFailed, notSupported,

QoSRequestFailed, noSuchFlow);

Videophone_StreamCtrl videophone_bind(

in videophone peer_device,

inout streamQoS the_qos,

out boolean is_met,

in flowSpec the_spec)

raises (streamOpDenied, streamOpFailed, notSupported);

Videophone_StreamCtrl videophone_bind_mcast(

in videophone first_peer,

inout streamQoS the_qos,

out boolean is_met,

in flowSpec the_spec)

raises (streamOpDenied, streamOpFailed, notSupported);

};

## The VDev

The VDev abstracts the idea of a multimedia device which can be linked up to peers across a network. This is the interface which requires most effort on the programmer’s part since the behaviour of every device will differ. The IDL for VDev is given below:

interface VDev : PropertyService::PropertySet{

boolean set_peer(

in StreamCtrl the_ctrl,

in VDev the_peer_dev,

inout streamQoS the_qos,

in flowSpec the_spec)

raises (noSuchFlow, QoSRequestFailed, streamOpFailed);

boolean set_Mcast_peer(in StreamCtrl the_ctrl,

in MCastConfigIf a_mcastconfigif,

inout streamQoS the_qos,

in flowSpec the_spec)

raises (noSuchFlow, QoSRequestFailed, streamOpFailed);

void configure(in PropertyService::Property the_config_mesg)

raises(PropertyException, streamOpFailed);

// Uses <formatName> standardised by OMG and IETF

void set_format(in string flowName, in string format_name)

raises (notSupported);

// Note, some of these device params are standardised by OMG

void set_dev_params(in string flowName,

in PropertyService::Properties new_params)

raises(PropertyException, streamOpFailed);

boolean modify_QoS(inout streamQoS the_qos,

in flowSpec the_spec)

raises (noSuchFlow, QoSRequestFailed);

};

The application developer or multimedia device vendor must implement the behaviour for set_peer() and set_Mcast_peer(). The purpose of these operations is to ensure that all flows originating in this VDev can be understood by the peer VDev. The programmer is free to implement this operation in any way he chooses. Typically the VDev will query the properties of its peer to see what formats it supports for its various flows. In cases where the Protection parameter is set in the QoS specification, it can also find out the public key for its peer flows. The configuration of the peer device will generally consist of calls to set_format() and set_dev_params() on the peer VDev or MCastConfigIf. Any type of configuration which can’t be achieved with these calls can be done through setting miscellaneous properties using the catch-all configure() calls to the peer device and in the multipoint case also calling set_initial_configuration().  The set_peer() call can also be used to get the public key of a peer device, this will be followed by a call to set_key() on the local StreamEndPoint.

The application programmer is free to reimplement the behaviour of modify_QoS(). This has already been discussed in Section 2.10.

The stream notation compiler generates the following videophone interface which specialises VDev:

interface v_Videophone : VDev{};

## The FlowEndPoint

The FlowEndPoint interface is only required to be supported in the full profile. The IDL for FlowEndPoint is:

interface FlowEndPoint : PropertyService::PropertySet

{

boolean lock();

void unlock();

void stop();

void start();

void destroy();

// Default is a nil object reference

attribute StreamEndPoint related_sep;

attribute FlowConnection related_flow_connection;

FlowEndPoint get_connected_fep()

raises (notConnected, notSupported);

// syntax of fp_name is <flowProtocol>

boolean use_flow_protocol(in string fp_name,

in any fp_settings)

raises (FPError, notSupported);

// set_format() initializes ‘format’

// as current media format e.g. MPEG.

void set_format(in string format)

raises (notSupported);

void set_dev_params(

in PropertyService::Properties new_settings)

raises (PropertyException, streamOpFailed);

void set_protocol_restriction(in protocolSpec the_spec)

raises (notSupported);

boolean is_fep_compatible(in FlowEndPoint fep)

raises (formatMismatch, deviceQosMismatch);

boolean set_peer(

in FlowConnection the_fc,

in FlowEndPoint the_peer_fep,

inout AVStreams::QoS the_qos)

raises (AVStreams::QoSRequestFailed,

AVStreams::streamOpFailed);

boolean set_Mcast_peer(

in FlowConnection the_fc,

in MCastConfigIf a_mcastconfigif,

inout QoS the_qos)

raises (QoSRequestFailed);

boolean connect_to_peer(inout AVStreams::QoS the_qos,

in string address,

in string use_flow_protocol) // syntax <flowProtocol>

raises(failedToConnect,

AVStreams::FPError, AVSTreams::QoSRequestFailed);

string go_to_listen(

inout AVStreams::QoS the_qos,

in boolean is_mcast,

in FlowEndPoint peer,

inout string flowProtocol)// syntax <flowProtocol>

// The out value contains SFP

// version supported and

// all options including

// “Credit”

raises(failedToListen, AVStreams::FPError,

AVStreams::QoSRequestFailed);

};

interface FlowProducer : FlowEndPoint

{

string connect_mcast(inout QoS the_qos, out boolean is_met,

in string address,

in string use_flow_protocol)

raises (failedToConnect, notSupported,

FPError, QoSRequestFailed);

string get_rev_channel(in string pcol_name);

void set_key(in key the_key);

void set_source_id(in long source_id);

};

interface FlowConsumer : FlowEndPoint

{

};

The lock() and unlock() operations reserve a FlowEndPoint for use with a particular flow. This would typically by called as a result of an add_fep() operation on a StreamEndPoint. Once a FlowEndPoint is locked, it cannot be locked by another StreamEndPoint until it has been unlocked by the current owner, i.e. the lock() call will return false. This problem does not arise if FDevs are used to create FlowEndPoints (which is recommended).

Most of the other operations will be familiar from StreamEndPoint and VDev. The is_fep_compatible() checks whether this fep supports compatible protocols and formats to its peer fep.

The connect_to_peer() operation connects one FlowEndPoint to its peer which has already been put into listen mode through a call to go_to_listen(). The get_rev_channel() call is made by a FlowConsumer on a FlowProducer where UDP-like unidirectional protocol has been chosen and SFP is being used. Since SFP sends messages in both directions, it needs a channel to send backward messages on such as Credit.

The connect_mcast() operation performs connection for multicast flows. A typical scenario is as follows. A call is made to add_producer() on the FlowConnection this will map to a call to connect_mcast() on a FlowProducer with an empty string for the address parameter. This call will return a string with the syntax <address> (see section ). If the connect_mcast() call returns a string with a full address (including protocol name, transport address and port number) then internet-style multicasting is in use. The returned string will contain the multicast address of the flow. Any subsequent calls to add_consumer() on the  FlowConnection will result in a call to go_to_listen() on the consumer with the multicast address as a parameter. If, on the other hand, a string was returned by the call to add_producer() which contains only the <carrier_protocol> part of the <address> syntax then ATM-style multicast is in use. Subsequent calls to add_consumer() will use set_protocol_restriction() calls on the consumer with the name of the returned carrier protocol e.g. “AAL5”. This will ensure that when go_to_listen() is called on the FlowConsumer (with is_mcast set to true) that it will be listening on the right protocol. The FlowConnection will then call connect_mcast() on the FlowProducer with listening address returned by go_to_listen().

A suitable stream notation compiler will produce the following interfaces for a flow of type X:

interface X_Producer : FlowProducer

{

};

interface X_Consumer : FlowConsumer

{

};

The IDL for MediaControl is:

enum PositionOrigin {

AbsolutePosition, RelativePosition, ModuloPosition

};

enum PositionKey {

ByteCount, SampleCount, MediaTime

};

struct Position {

PositionOrigin origin;

PositionKey key;

long value;

};

exception PostionKeyNotSupported { PositionKey key;};

exception InvalidPosition { PositionKey key;};

exception InvalidTransform {};

// MediaControl interface is similar to

// ControlledStream interface in MSS.

// It can be inherited by flow endpoints or

// FlowConnection interfaces.

interface MediaControl{

exception PostionKeyNotSupported { PositionKey key;};

Position get_media_position(

in PositionOrigin an_origin,

in PositionKey a_key)

raises (PostionKeyNotSupported);

void set_media_position(in Position a_position)

raises (PostionKeyNotSupported, InvalidPosition);

void start(in Position a_position)

raises(InvalidPosition);

void pause(in Position a_position)

raises(InvalidPosition);

void resumes(in Position a_position)

raises(InvalidPosition);

void stop(in Position a_position)

raises(InvalidPosition);

};

A MediaCtrl interface can be associated with a VDev (using the FlowNameX_related_mediaCtrl property) or FlowEndPoint (using the Related_mediaCtrl property) and it may be multiply inherited by specialisations of FlowConnection or StreamCtrl. For example:

interface myStreamCtrl : StreamCtrl, MediaControl{};

Instances of  myStreamCtrl allow standardised control over the stored media replay. The importance of this type of scenario merits the inclusion of this interface as standard. Appendix C on the relationship to DAVIC also deals with this subject.

## The FlowConnection

This interface is only implemented by the full profile.

interface FlowConnection : PropertyService::PropertySet{

void stop();

void start();

void destroy();

boolean modifyQoS(inout AVStreams::QoS new_qos)

raises (AVStreams::QoSRequestFailed);

boolean use_flow_protocol(

in string fp_name,

in any fp_settings)

raises (FPError, notSupported);

oneway void push_event(in streamEvent the_event);

boolean connect_devs(in FDev a_party, in FDev b_party,

inout QoS the_qos);

boolean connect(

in FlowProducer flow_producer,

in FlowConsumer flow_consumer,

inout QoS theQoS)

raises (formatMismatch, FEPMismatch, alreadyConnected);

boolean disconnect();

// The notSupported exception is raised where

// flow cannot have multiple producers

boolean add_producer(in FlowProducer flow_producer,

inout QoS theQoS)

raises (alreadyConnected, notSupported);

boolean add_consumer(in FlowConsumer flow_consumer,

inout QoS theQoS)

raises (alreadyConnected);

boolean drop(in FlowEndPoint target)

raises (notConnected);

};

The FlowConnection is the flow-level analogue of the StreamCtrl. A full profile implementation of the StreamCtrl which connects two or more stream endpoints which use a flow connection for each of the individual flows within a stream.

The following paragraph describes the typical sequence of events in using a full profile StreamCtrl to bind to two full profile stream endpoints:

User A adds some FEPs to an instance of a full profile implementation of StreamEndPoint_A (myA) by calling myA->add_fep(aFEP);

User B adds some FEPs to an instance of a full profile implementation of StreamEndPoint_B (theirB).

User C creates a StreamCtrl and calls

aSC->bind(myA,theirB,someQoS,nilflowSpec)

The bind() algorithm will find which pairs of FlowEndPoints are compatible between myA and theirB and create a FlowConnection for each pair of FlowEndPoints. Each flow connection within a stream can be individually accessed and manipulated.

A suitable stream notation compiler can generate the following code for a flow of type X:

interface X_Connection : FlowConnection {

boolean connect_X_devs(in F_X a_party, in F_X b_party,

inout QoS the_qos);

boolean connect_X(

in X_Producer flow_producer,

in X_Consumer flow_consumer,

inout QoS theQoS)

raises (formatMismatch, FEPMismatch, alreadyConnected);

// The notSupported exception is raised where

// flow cannot have multiple producers

boolean add_X_producer(in X_Producer flow_producer,

inout QoS theQoS)

raises (alreadyConnected, notSupported);

boolean add_X_consumer(in X_Consumer flow_consumer,

inout QoS theQoS)

raises (alreadyConnected);

};

## FDev

The FDev is only used in the full profile. The IDL is given below:

interface FDev : PropertyService::PropertySet {

FlowProducer create_producer(

in FlowConnection the_requester,

inout QoS the_qos,

out boolean met_qos,

inout string named_fdev)

raises(streamOpFailed, streamOpDenied, notSupported,

QoSRequestFailed);

FlowConsumer create_consumer(

in FlowConnection the_requester,

inout QoS the_qos,

out boolean met_qos,

inout string named_fdev)

raises(streamOpFailed, streamOpDenied, notSupported,

QoSRequestFailed);

FlowConnection bind(in FDev peer_device,

inout QoS the_qos,

out boolean is_met)

raises (streamOpFailed, QoSRequestFailed);

FlowConnection bind_mcast(in FDev first_peer,

inout QoS the_qos,

out boolean is_met)

raises (streamOpFailed, QoSRequestFailed);

void destroy(in FlowEndPoint the_ep, in string fdev_name)

// ie FDev not found

raises (notSupported);

};

The FDev is exactly analogous in operation to the MMDevice for streams.

An stream notation compiler could generate the following specialisation of FDev for a flow type X:

interface F_X : FDev {

X_Producer create_X_producer(

in X_Connection the_requester,

inout QoS the_qos,

out boolean met_qos,

inout string named_fdev)

raises(streamOpFailed, streamOpDenied, notSupported,

QoSRequestFailed);

X_Consumer create_X_consumer(

in X_FlowConnection the_requester,

inout QoS the_qos,

out boolean met_qos,

inout string named_fdev)

raises(streamOpFailed, streamOpDenied, notSupported,

QoSRequestFailed);

X_Connection X_bind(in F_X peer_device,

inout QoS the_qos,

out boolean is_met)

raises (streamOpFailed, QoSRequestFailed);

X_Connection X_bind_mcast(in F_X first_peer,

inout QoS the_qos,

out boolean is_met)

raises (streamOpFailed QoSRequestFailed);

};

# Conformance Criteria

This section summarises the various levels of conformance to the specification.

## Light vs Full Profile

There are two main levels of specification: “light” and “full”. The “light” implementation must support the following interfaces, as described in the text:

PropertyService::PropertySet, Basic_StreamCtrl, StreamCtrl, Negotiator, MCastConfigIf, StreamEndPoint, StreamEndPoint_A, StreamEndPoint_B, VDev, MMDevice.

In addition to these, the full profile must support the following interfaces, as described in the text:

FlowConnection, FlowEndPoint, FlowConsumer, FlowProducer, FDev, MediaControl.

The following operations in the light profile will raise the notSupported exception with reason “Full profile not supported”:

In StreamEndPoint: get_fep(), add_fep(), remove_fep()

In Basic_StreamCtrl: get_flow_connection(), set_flow_connection()

## Flow Protocol

Both light and full profiles may optionally support a flow protocol. SFP version 1.0 is specified in this document. A system which does not support SFP causes the exception FPError with reason “No flow protocol  supported”. If the version of SFP supported is different to that requested, then it uses reason “<flowProtocol> only supported”. The following operations raise the FPError exception:

In Basic_StreamCtrl: set_FPStatus()

In StreamEndPoint: set_FPStatus(), request_connection()

In FlowConnection: use_flow_protocol()

In FlowEndPoint: use_flow_protocol()

## Network QoS Parameters

It is mandatory for a streams implementation to support Network QoS parameter set 1 (as discussed in section 2.13). It is optional to support Network QoS parameter set 2.

# Appendix A - Meeting RFP Requirements for Control and Management of A/V Streams

This section discusses how the issues raised in the RFP for Control and Management of A/V Streams are addressed by the architecture outlined in this document. Please note that all IDL in this document has been compiled by the Orbix 2.2 IDL compiler on NT and product development work is underway. This development work includes a running prototype for untyped streams.

## Topologies

The RFP requires a solution to: “allow one-to-one, one-to-many, many-to-one and many-to-many sources and sinks to be configured in the same stream binding”. The architecture presented in this submission addresses point-to-point and point-to-multipoint configurations which can easily be used as the building blocks of many-to-many and many-to-one streams. For example, the application developer or third party supplier could define an interface manyPt_Stream which inherits from Basic_StreamCtrl and which supports an operation join_party(in MMDevice newparty). The programmer may then choose to implement many-to-many streams using a ‘video-bridge’ style device. This is illustrated below:

Figure 10 - Using a mixer device for creating multipoint-to-multipoint streams

When the application programmer calls to stop() on manyPt_streamCtrl this will result in stop() being called on all constituent streams.

Figure 11 - Using internet multicasting to implement multipoint-to-multipoint streams

Using internet multicasting to support many-to-many streams (e.g. Figure 11) is even easier. The application programmer uses a call to set_FPStatus() to indicate that source-ids should be used. By configuring his MediaStreams implementation he can ensure that UDP is the default protocol. He subsequently calls:

bind_devs(Dev1,nilObject,…)

bind_devs(nilObject,Dev1,…)

bind_devs(Dev2,nilObject,…)

bind_devs(nilObject,Dev2,…)

bind_devs(Dev3,nilObject,…)

bind_devs(nilObject,Dev3,…)

All StreamEndPoints will receive data from all other stream endpoints. They will use source-ids to distinguish which frames are from which endpoints. The application must then perform its own ‘mixing’ of the received data similar to other internet conferencing software.

The important point is that this submission allows multipoint-to-multipoint streams to be built but does not force the use of any one approach. This is a very important area where vendors and third parties can add value.

## Multiple Flows

Multiple flows are an integral part of the solution presented in this submission. A stream may contain any number of flows and an instruction to stop or start can be applied to the stream as a whole or any subset of its flows. A stream endpoint contains a FlowEndPoint object for each flow within the stream. In the “light” version of the specification however, FlowEndPoints do not expose their IDL. In the light profile, flows information is instead accessed  via the StreamEndPoint interfaces.

## Stream Description and Typing

It is beneficial for a stream service implementation to provide a notation to specify the content of flows and their relative direction within a particular stream type. This notation, however, is beyond the scope of this RFP response. A de facto notation such as TINA-C ODL stream template, for example, could be used. It is expected that another RFP will be issued which will cover the area of stream typing notation and the related language mappings for typesafe insertion and extraction of data to from a flow.

Although the notation for typing streams is beyond the scope of this document nevertheless, SFP provides for transportation of IDL typed data. This RFP has also described a scheme for creating strictly typed versions of the interfaces for StreamCtrl, MMDevice, VDev, StreamEndPoint_A, StreamEndPoint_B, FlowConnection, FlowProducer, FlowConsumer and FDev. This allows typed streams to be dealt with in a standardised way without dictating the notation used to describe streams or the language mappings for inserting/extracting typed information to/from flows.

## Stream Interface Identification and Reference

A reference to a stream is simply a reference to a StreamCtrl interface. A reference to a stream endpoint is a reference to a StreamEndPoint.

## Stream setup and release

Stream setup and release are the main focus of this submission. Streams can be established flexibly using either the MMDevice::bind() operations or the bind_devs(), bind() family of operations in StreamCtrl. There is also support for releasing stream connections using unbind(), destroy() or unbind_party(). Individual flows may also be setup and released in both the light and full profiles.

## Stream Modification

One of the principal advantages of streams is that they can be flexibly modified during their lifetime. These modifications include changes to QoS using the modify_QoS()operations. An appropriate error is thrown whenever a QoS modification fails. It should be recognised that changes to application level QoS cannot be made in isolation of the VDevs being linked by a stream. A VDev may reject the QoS modification because it would overload the underlying multimedia device.

All streams can be modified during their lifetime through the addition or removal of individual flows.

Multipoint streams add the flexibility of being able to add or remove A or B parties.

## Stream Termination

The RFP specifies that it should be possible to terminate flow-endpoints “in hardware or software”. This can be achieved using the architecture outlined in this submission by suitable implementation of the StreamEndPoint or FlowEndPoint. For example, a StreamEndPoint_A or StreamEndPoint_B derived object which overrides behaviours in the normal StreamEndPoint_A and StreamEndPoint_B could execute methods  which signal directly to hardware devices on the network. The StreamCtrl therefore need not be aware which flows are terminated in hardware and which are terminated in software.

## Multiple Protocols

The submission was designed with multiple protocols in mind. This will allow the solution to be applied, in principle, to any transport protocol, including ATM protocols and internet protocols. The response makes explicit provision for RTP. The protocols to be used for carrying individual streams will be selected at runtime in the process of establishing a connection. The stream endpoint may be restricted in the choice of protocol by explicitly setting a protocol restriction (set_protocol_restriction()). This can be used to prevent an A party from choosing protocols which a B party does not speak. The details of how different transports are abstracted internally to the end-points is an implementation issue.

## Flow Synchronisation

Inter-flow synchronisation is not tackled within a stream. The position of this submission is that inter-flow synchronisation is more properly an issue to be dealt with by the device endpoints. The SFP includes a timestamp field which can be used by the VDev implementation at the receiving end to perform synchronisation between different flows by reconstructing timing relationships at the source.

## Interoperability

Interoperability in this submission has a number of different aspects:

- Naming of generated interfaces

- Compatibility of StreamEndPoint implementations

- Compatibility of SFP

- Compatability of common parameters, properties and syntaxes

The last point is dealt with in Section 2.13 and in the A/V Streams registration space which is presented in Section 5.12. The syntaxes for common string parameters such as <flowSpec> are also defined in Section 3 of this document.

### Naming of Generated Interfaces

These interoperability rules concern the way that a stream notation compiler will generate IDL interfaces. The rules are relatively simple and should be respected by all conforming stream-IDL compilers regardless of the notation which they use to represent streams.

If a stream interface is named X then the generated IDL interfaces will be:

- X_StreamCtrl derived from StreamCtrl

- X_A and X_B - Derived from StreamEndPoint_A and StreamEndPoint_B

- X - Derived from MMDevice

- v_X - Derived from VDev

If a flow type is named Y then the generated IDL interfaces will be:

- Y_FlowConnection derived from FlowConnection

- Y_Consumer and Y_Producer - derived from FlowConsumer and FlowProducer

- F_Y - Derived from FDev

A full description of all the IDL generated for each of these is given in section 3.

### Compatibility of StreamEndPoints

It should be possible to set up a stream between two device objects even if the two device objects belong to two different implementations of the streaming service. This requires that implementations offer uniform behaviour. This is particularly important between StreamEndPoint objects in the “light” profile of the service. In particular the connect() call on the “light” StreamEndPoint must follow a particular sequence of calls to its peer StreamEndPoint. This sequence of operations between the “light” versions of the end-points is detailed in section 3.4.

### Compatibility of Flow Formats

Compatability of flow data is not an issue where raw data in an agreed byte layout is being exchanged (e.g. a raw MPEG flow). At a higher level the architecture provides for extensive negotiation between VDevs prior to stream setup which ensures that the source of a flow is producing information in a format which is understanding to the sink of the flow. This VDev negotiation is one of the most important aspects of this submission.

Where typed data needs to be transferred or where in-band information such as timestamping is needed, compatibility of flows formats will be assured through the (optional) use of the SFP defined in Section 2. Figure 12 below depicts the state machine for the sending end of an SFP flow in the point-to-point case. Figure 13 depicts the state machine for the receiving end in the point-to-point case. The multipoint case is of course much simpler since no information ever travels in the reverse direction. Each transition is labelled by the condition which causes the transition to occur and the action which is performed when the transition is made. Note that the state machines have been slightly simplified in that the sending of fragments is not shown. Also, the machines implementing the protocol at each end will dicard any messages which are received when they are not expected: e.g. if the sender gets a credit when it is not expected it will be discarded. The timeout value T2 should be set to be several times the duration of timeout of T1. This allows for several attempts by the other side to send their message.

Figure 12 - The state machine for the SFP sender

Figure 13 - The state machine for the SFP receiver

## Security

One of the principal advantages of using a distributed streaming framework is that it allows uniform application of security policies to streamed media. Regular security mechanisms can be applied to operations invoked on any of the objects in the media streams framework. The mechanisms for encryption are beyond the scope of this RFP response, however the hooks provided by the interfaces should allow encryption to be applied to selected flows within a stream. The “Protection” attribute is a registered QoS parameter. In order to protect a flow the application programmer need only specify that the “Protection” QoS parameter is set to 1 (for encryption). The set_key() operation on the StreamEndPoint and FlowEndPoint then allows the flow/flows to be established or modified to use either public or private key encryption. Where public key encryption is to be used, Public keys can be exchanged via VDev negotiation i.e. the VDev finds the value of FlowNameX_PublicKey on its peer and calls the set_key()operation with this value.

## The A/V Streams Registration Space

The A/V Streams initial registration space is given below. This information will be maintained in a living document under the OMG’s Website. The A/V Streams registration space web page will contain instructions on how to register new values.

### Carrier Protocol Names

<CarrierName> ::=

"TCP" |

"UDP" |

"AAL5" |

"IPX" |

"RTP/UDP" |

“RTP/AAL5”|

“AAL3-4” | “AAL1”

Other names may be registered via the OMG registration authority.

### SFP

The <flowProtocol> syntax currently allows the following combinations of values:

<flowProtocolName> ::= “SFP”

The <version> for SFP is currently “1.0”.

The only currently defined <OptionTag> is “Credit” which takes a non-zero positive integer <OptionValue>

### Media Format Categories and MIME Content-Types

<FormatCategory> ::=

"IDL" |

"MIME"

The Media-type registry for audio and video subtypes is currently very sparse. For the purposes of this RFP submission we will temporarily define the following Media-types. These will be officially registered with the IETF in due course:

video/x-mjpeg, the MJPEG standard for video

The following media-types are also supported.

audio/mpeg

audio/x-wav, the Microsoft audio format

audio/x-aiff

video/x-msvideo, the Microsoft format, e.g. .avi files

video/x-mpeg2

### QoS Parameters

Names for QoS parameter types and their associated IDL types

struct resolution{

long horz,

long vert,

};

"audio_sampleSize" - short, number of bits per sample

"audio_sampleRate" - long, Hertz

"audio_numChannels" - short

"audio_quantisation" - short, 0 = linear, 1 = u-law, 2 = A-law, 3 = GSM

"video_framerate" - long

"video_colourDepth" - short, e.g. 2, 4, 8, 12, 16, 24, 32

"video_colourModel" - short 0 - RGB, 1 - YUV, 2 - HSV

"video_resolution" - struct resolution.

The following parameters are also standardised for Network-level QoS (QoSType = “Network_QoS”)

Parameter set 1 - Support = Mandatory

- “ServiceType” - short, 0 = best effort, 1 = guaranteed, 2 = predicted

- “ErrorFree” - boolean

- “Delay” - long, milliseconds, desired delay

- “Delay_Max” - long, max. acceptable delay

- “Bandwidth” - long, bytes per second, desired bandwidth

- “Bandwidth_Min” - long, bytes per second, min. acceptable bandwidth

- “Bandwidth_Max” - long, max. offered bandwidth

- “PeakBandwidth” - long, bytes per second, desired peak bandwidth

- “PeakBandwidth_Min” - long, min. acceptable peak bandwidth

- “PeakBandwidth_Max” - long, max. offered bandwidth

- “TokenRate” - long value, bytes/second

- “TokenRate_Min” - long value

- “TokenRate_Max” - long value

- “TokenBucketSize” - long value, bytes

- “TokenBucketSize_Min” - long value

- “TokenBucketSize_Max” - long value

- “Jitter” - float, microseconds, bounds for delay variation

- “Jitter_Max” - float, max. acceptable

- “Cost” - float, dimensionless

- “Cost_Max” - float, max. acceptable cost

- “Protection” - short, 0= default, no encryption, 1= encryption level 1

Parameter set 2 - Support = Optional

- Duplication - enum dup {IGNORE, DELETE}

- Damage - enum dam {DAM_IGNORE, DAM_NOTIFY, DAM_DELETE, DAM_CORRECT}

- Damage_method - Type to be specified

- Reorder - enum reord {REORDER_CORRECT, REORDER_IGNORE}

- Loss - long, {-1 = LOSS_IGNORE, -2 = LOSS_NOTIFY}, positive integer denotes number of retry attempts before the receiver is presumed dead

- Size_Min - long, min bytes in a data unit

- Size_Max - long, max bytes in a data unit

- Size_avg - long, average number of bytes in a data unit

- Size_avg_span - long, number of subsequent data units sent with an interval (ival_const)

- Ival_Const - long, {-2 = IVAL_MAX, -1 = IVAL_ANY}, positive integer denotes constant time interval between transport requests

- Ival_Max - long, the maximum acceptable value if Ival_Const cannot be met

- Delay - long, {0 = DELAY_VOID, -1 = DELAY_ANY, -2 = DELAY_MIN}, DELAY_MIN denotes best effort with minimal delay, DELAY_ANY = best effort, low cost. Positive integer denotes delay required.

- Delay_Max - long, indicates maximum acceptable delay

- Delay_Cum - long, indicates acceptable cumulative delay for concatenated stream

- Jitter - long

- Jitter_Max - long

- ErrDamRatio - float, Ratio of damaged data units {0.0 = RATIO_DAM_VOID, -1.0 = RATIO_DAM_ANY, -2.0 = RATIO_DAM_MIN}, where RATIO_DAM_MIN is a request for minimal error ratio, RATIO_DAM_ANY is request for less costly ratio. A number between 0-1.0 indicates the desired ratio

- ErrDamRatio_Max - float, maximum acceptable error ratio for damaged data units

- ErrLossRatio - float, ratio of lost data units { 0.0 = RATIO_LOSS_VOID, -1.0 = RATIO_LOSS_ANY, -2,0 = RATIO_LOSS_MIN}, where RATIO_LOSS_MIN is a request for minimal error ratio, RATIO_LOSS_ANY is request for less costly ratio. A number between 0-1.0 indicates the desired ratio

- ErrLossRatio_Max - float, maximum acceptable ratio of lost data units

- Workahead_Mode - enum {AHEAD_BLOCKING, AHEAD_NONBLOCKING}

- Workahead_Max - long, the maximum number of data units the producer may be ahead of the consumer

- Playback_Mode - Type to be specified, indicates playback strategy

- Playback_Max - long, The maximum delay introduced by the producer to counter jitter effects

### Device Parameters

The following device parameters are standardised by this document. Similarities to QoS parameters are non-accidental J.

“language” - string, from the set {…,“English(UK)”,”English(US)”,…,”Irish”,…}

"audio_sampleSize" - short, number of bits per sample

"audio_sampleRate" - long, Hertz

"audio_numChannels" - short

"audio_quantisation" - short, 0 = linear, 1 = u-law, 2 = A-law, 3 = GSM

"video_framerate" - long

"video_colourDepth" - short, e.g. 2, 4, 8, 12, 16, 24, 32

"video_colourModel" - short 0 - RGB, 1 - YUV, 2 - HSV

"video_resolution" - struct resolution.

# Appendix B - Full IDL

This gives full IDL for the Errata on the Revised Submission. All IDL compiles using the Orbix 2.2 IDL compiler on NT.

#include "PropertyService.idl"

module AVStreams{

struct QoS{

string QoSType;

PropertyService::Properties QoSParams;

};

typedef sequence<QoS> streamQoS;

typedef sequence<string> flowSpec;

typedef sequence<string> protocolSpec;

typedef sequence<octet> key;

// protocol names registered by OMG.

// e.g., TCP, UDP, AAL5, IPX, RTP

// This structure is defined for SFP1.0

// Subsequent versions of the protocol may

// specify new structures

struct SFPStatus{

boolean isFormatted;

boolean isSpecialFormat;

boolean seqNums;

boolean timestamps;

boolean sourceIndicators;

};

enum flowState {stopped, started, dead};

enum dirType {dir_in, dir_out};

struct flowStatus{

string flowName;

dirType directionality;

flowState status;

SFPStatus theFormat;

QoS theQoS;

};

typedef PropertyService::Property streamEvent;

exception notSupported {};

exception PropertyException {};

// An flow protocol related error

exception FPError { string flow_name; };

exception streamOpFailed{

string reason;};

exception streamOpDenied{

string reason;};

exception noSuchFlow{};

exception QoSRequestFailed{

string reason;};

interface Basic_StreamCtrl : PropertyService::PropertySet {

// Empty flowSpec => apply operation to all flows

void stop(in flowSpec the_spec) raises (noSuchFlow);

void start(in flowSpec the_spec) raises (noSuchFlow);

void destroy(in flowSpec the_spec) raises (noSuchFlow);

boolean modify_QoS(inout streamQoS new_qos,

in flowSpec the_spec)

raises (noSuchFlow, QoSRequestFailed);

// Called by StreamEndPoint when something goes wrong

// with a flow

oneway void push_event(

in streamEvent the_event);

void set_FPStatus(in flowSpec the_spec,

in string fp_name,		// Only SFP1.0 currently

// specified

in any fp_settings)		// Currently SFP accepts

// SFPStatus structure

raises (noSuchFlow, FPError);

Object get_flow_connection(in string flow_name)

raises (noSuchFlow, notSupported);

void set_flow_connection(in string flow_name,

in Object flow_connection)

raises (noSuchFlow, notSupported);

};

interface Negotiator{

boolean negotiate(in Negotiator remote_negotiator,

in streamQoS qos_spec);

};

interface VDev;

interface MMDevice;

interface StreamEndPoint;

interface StreamEndPoint_A;

interface StreamEndPoint_B;

interface StreamCtrl : Basic_StreamCtrl {

boolean bind_devs(in MMDevice a_party, in MMDevice b_party,

inout streamQoS the_qos,

in flowSpec the_flows)

raises (streamOpFailed, noSuchFlow, QoSRequestFailed);

boolean bind(in StreamEndPoint_A a_party,

in StreamEndPoint_B b_party,

inout streamQoS the_qos,

in flowSpec the_flows)

raises (streamOpFailed, noSuchFlow, QoSRequestFailed);

void unbind_dev(in MMDevice dev,

in flowSpec the_spec)

raises (streamOpFailed, noSuchFlow);

void unbind_party(in StreamEndPoint the_ep,

in flowSpec the_spec)

raises (streamOpFailed, noSuchFlow);

void unbind()

raises (streamOpFailed);

VDev get_related_vdev(in MMDevice adev,

out StreamEndPoint sep)

raises (streamOpFailed);

};

interface MCastConfigIf : PropertyService::PropertySet{

boolean set_peer(

in Object peer,

inout streamQoS the_qos,

in flowSpec the_spec)

raises (QoSRequestFailed, streamOpFailed);

void configure(in PropertyService::Property a_configuration);

void set_initial_configuration(

in PropertyService::Properties initial);

// Uses <format_name> standardised by OMG and IETF

void set_format(in string flowName, in string format_name)

raises (notSupported);

// Note, some of these device params are standardised by OMG

void set_dev_params(in string flowName,

in PropertyService::Properties new_params)

raises(PropertyService::PropertyException,

streamOpFailed);

};

interface StreamEndPoint : PropertyService::PropertySet{

void stop(in flowSpec the_spec) raises (noSuchFlow);

void start(in flowSpec the_spec) raises (noSuchFlow);

void destroy(in flowSpec the_spec) raises (noSuchFlow);

boolean connect(in StreamEndPoint responder,

inout streamQoS qos_spec,

in flowSpec the_spec)

raises (noSuchFlow, QoSRequestFailed, streamOpFailed);

boolean request_connection(

in StreamEndPoint initiator,

in boolean is_mcast,

inout streamQoS qos,

inout flowSpec the_spec)

raises (streamOpDenied, noSuchFlow,

QoSRequestFailed, FPError);

boolean modify_QoS(inout streamQoS new_qos,

in flowSpec the_flows)

raises (noSuchFlow, QoSRequestFailed);

boolean set_protocol_restriction(in protocolSpec the_pspec);

void disconnect(in flowSpec the_spec)

raises (noSuchFlow, streamOpFailed);

void set_FPStatus(in flowSpec the_spec,

in string fp_name,

in any fp_settings)

raises (noSuchFlow, FPError);

Object get_fep(in string flow_name)

raises (notSupported, noSuchFlow);

string add_fep(in Object the_fep)

// Can fail for reasons {duplicateFepName, duplicateRef}

raises (notSupported, streamOpFailed);

void remove_fep(in string fep_name)

raises (notSupported, streamOpFailed);

void set_negotiator(in Negotiator new_negotiator);

void set_key(in string flow_name, in key the_key);

void set_source_id(in long source_id);

};

interface StreamEndPoint_A : StreamEndPoint{

boolean multiconnect(inout streamQoS the_qos,

inout flowSpec the_spec)

raises (noSuchFlow, QoSRequestFailed, streamOpFailed);

boolean connect_leaf(in StreamEndPoint_B the_ep,

inout streamQoS the_qos,

in flowSpec the_flows)

raises (streamOpFailed, noSuchFlow,

QoSRequestFailed, notSupported);

void disconnect_leaf(in StreamEndPoint_B the_ep,

in flowSpec theSpec)

raises(streamOpFailed, noSuchFlow);

};

interface StreamEndPoint_B : StreamEndPoint {

boolean multiconnect(inout streamQoS the_qos,

inout flowSpec the_spec)

raises (streamOpFailed, noSuchFlow,

QoSRequestFailed, FPError);

};

interface VDev : PropertyService::PropertySet{

boolean set_peer(

in StreamCtrl the_ctrl,

in VDev the_peer_dev,

inout streamQoS the_qos,

in flowSpec the_spec)

raises (noSuchFlow, QoSRequestFailed, streamOpFailed);

boolean set_Mcast_peer(in StreamCtrl the_ctrl,

in MCastConfigIf a_mcastconfigif,

inout streamQoS the_qos,

in flowSpec the_spec)

raises (noSuchFlow, QoSRequestFailed, streamOpFailed);

void configure(in PropertyService::Property the_config_mesg)

raises(PropertyException, streamOpFailed);

// Uses <formatName> standardised by OMG and IETF

void set_format(in string flowName, in string format_name)

raises (notSupported);

// Note, some of these device params are standardised by OMG

void set_dev_params(in string flowName,

in PropertyService::Properties new_params)

raises(PropertyException, streamOpFailed);

boolean modify_QoS(inout streamQoS the_qos,

in flowSpec the_spec)

raises (noSuchFlow, QoSRequestFailed);

};

interface MMDevice : PropertyService::PropertySet {

StreamEndPoint_A create_A(

in StreamCtrl the_requester,

out VDev the_vdev,

inout streamQoS the_qos,

out boolean met_qos,

inout string named_vdev,

in flowSpec the_spec)

raises(streamOpFailed, streamOpDenied, notSupported,

QoSRequestFailed, noSuchFlow);

StreamEndPoint_B create_B(

in StreamCtrl the_requester,

out VDev the_vdev,

inout streamQoS the_qos,

out boolean met_qos,

inout string named_vdev,

in flowSpec the_spec)

raises(streamOpFailed, streamOpDenied, notSupported,

QoSRequestFailed, noSuchFlow);

StreamCtrl bind(in MMDevice peer_device,

inout streamQoS the_qos,

out boolean is_met,

in flowSpec the_spec)

raises (streamOpFailed, noSuchFlow, QoSRequestFailed);

StreamCtrl bind_mcast(in MMDevice first_peer,

inout streamQoS the_qos,

out boolean is_met,

in flowSpec the_spec)

raises (streamOpFailed, noSuchFlow, QoSRequestFailed);

void destroy(in StreamEndPoint the_ep, in string vdev_name)

// ie VDev not found

raises (notSupported);

string add_fdev(in Object the_fdev)

raises(notSupported, streamOpFailed);

Object get_fdev(in string flow_name)

raises(notSupported, noSuchFlow);

void remove_fdev(in string flow_name)

raises (notSupported, noSuchFlow);

};

};

// Additional IDL for full profile

#include "AVStreams.idl"

module AVStreams_Full{

exception protocolNotSupported{};

exception formatNotSupported{};

exception formatMismatch{};

exception FEPMismatch{};

exception alreadyConnected{};

exception invalidSettings{string settings;};

exception notConnected{};

exception deviceQosMismatch{};

exception failedToConnect{string reason;};

exception failedToListen{string reason;};

interface FlowProducer;

interface FlowConsumer;

interface FlowEndPoint;

interface FDev;

interface FlowConnection : PropertyService::PropertySet{

void stop();

void start();

void destroy();

boolean modify_QoS(

inout AVStreams::QoS new_qos)

raises (AVStreams::QoSRequestFailed);

boolean use_flow_protocol(

in string fp_name,

in any fp_settings)

raises (AVStreams::FPError, AVStreams::notSupported);

oneway void push_event(in AVStreams::streamEvent the_event);

boolean connect_devs(in FDev a_party, in FDev b_party,

inout AVStreams::QoS the_qos)

raises (AVStreams::streamOpFailed,

AVStreams::streamOpDenied,

AVStreams::QoSRequestFailed);

boolean connect(

in FlowProducer flow_producer,

in FlowConsumer flow_consumer,

inout AVStreams::QoS the_qos)

raises (formatMismatch, FEPMismatch, alreadyConnected);

boolean disconnect();

// The notSupported exception is raised where

// flow cannot have multiple producers

boolean add_producer(in FlowProducer flow_producer,

inout AVStreams::QoS the_qos)

raises (alreadyConnected, AVStreams::notSupported);

boolean add_consumer(in FlowConsumer flow_consumer,

inout AVStreams::QoS the_qos)

raises (alreadyConnected);

boolean drop(in FlowEndPoint target)

raises (notConnected);

};

interface FlowEndPoint : PropertyService::PropertySet

{

boolean lock();

void unlock();

void stop();

void start();

void destroy();

// Default is a nil object reference

attribute AVStreams::StreamEndPoint related_sep;

attribute FlowConnection related_flow_connection;

FlowEndPoint get_connected_fep()

raises (notConnected,

AVStreams::notSupported);

// syntax of fp_name is <flowProtocol>

boolean use_flow_protocol(in string fp_name,

in any fp_settings)

raises (AVStreams::FPError, AVStreams::notSupported);

// set_format() initializes ‘format’

// as current media format e.g. MPEG.

void set_format(in string format)

raises (AVStreams::notSupported);

void set_dev_params(

in PropertyService::Properties new_settings)

raises (PropertyService::PropertyException,

AVStreams::streamOpFailed);

void set_protocol_restriction(in AVStreams::protocolSpec

the_spec)

raises (AVStreams::notSupported);

boolean is_fep_compatible(in FlowEndPoint fep)

raises (formatMismatch, deviceQosMismatch);

boolean set_peer(

in FlowConnection the_fc,

in FlowEndPoint the_peer_fep,

inout AVStreams::QoS the_qos)

raises (AVStreams::QoSRequestFailed,

AVStreams::streamOpFailed);

boolean set_Mcast_peer(

in FlowConnection the_fc,

in AVStreams::MCastConfigIf a_mcastconfigif,

inout AVStreams::QoS the_qos)

raises (AVStreams::QoSRequestFailed);

boolean connect_to_peer(inout AVStreams::QoS the_qos,

in string address,

in string use_flow_protocol) // syntax <flowProtocol>

raises(failedToConnect,

AVStreams::FPError, AVStreams::QoSRequestFailed);

// Needs to know its peer to choose its protocol correctly

// Also to ask for a reverse channel for credit-based flow

// control, if one is required

string go_to_listen(

inout AVStreams::QoS the_qos,

in boolean is_mcast,

in FlowProducer peer,

inout string flowProtocol)// syntax <flowProtocol>

raises(failedToListen, AVStreams::FPError,

AVStreams::QoSRequestFailed);

};

interface FlowProducer : FlowEndPoint

{

string connect_mcast(inout AVStreams::QoS the_qos,

out boolean is_met,

in string address,

in string use_flow_protocol)

raises (failedToConnect,

AVStreams::notSupported,

AVStreams::FPError,

AVStreams::QoSRequestFailed);

string get_rev_channel(in string pcol_name);

void set_key(in AVStreams::key the_key);

void set_source_id(in long source_id);

};

interface FlowConsumer : FlowEndPoint

{

};

interface FDev : PropertyService::PropertySet {

FlowProducer create_producer(

in FlowConnection the_requester,

inout AVStreams::QoS the_qos,

out boolean met_qos,

inout string named_fdev)

raises(AVStreams::streamOpFailed,

AVStreams::streamOpDenied,

AVStreams::notSupported,

AVStreams::QoSRequestFailed);

FlowConsumer create_consumer(

in FlowConnection the_requester,

inout AVStreams::QoS the_qos,

out boolean met_qos,

inout string named_fdev)

raises(AVStreams::streamOpFailed,

AVStreams::streamOpDenied,

AVStreams::notSupported,

AVStreams::QoSRequestFailed);

FlowConnection bind(in FDev peer_device,

inout AVStreams::QoS the_qos,

out boolean is_met)

raises (AVStreams::streamOpFailed,

AVStreams::QoSRequestFailed);

FlowConnection bind_mcast(in FDev first_peer,

inout AVStreams::QoS the_qos,

out boolean is_met)

raises (AVStreams::streamOpFailed,

AVStreams::QoSRequestFailed);

void destroy(in FlowEndPoint the_ep, in string fdev_name)

// ie FDev not found

raises (AVStreams::notSupported);

};

enum PositionOrigin {

AbsolutePosition, RelativePosition, ModuloPosition

};

enum PositionKey {

ByteCount, SampleCount, MediaTime

};

struct Position {

PositionOrigin origin;

PositionKey key;

long value;

};

exception PostionKeyNotSupported { PositionKey key;};

exception InvalidPosition { PositionKey key;};

// MediaControl interface is similar to

// ControlledStream interface in MSS.

// It can be inherited by flow endpoints or

// FlowConnection interfaces.

interface MediaControl{

exception PostionKeyNotSupported { PositionKey key;};

Position get_media_position(

in PositionOrigin an_origin,

in PositionKey a_key)

raises (PostionKeyNotSupported);

void set_media_position(in Position a_position)

raises (PostionKeyNotSupported, InvalidPosition);

void start(in Position a_position)

raises(InvalidPosition);

void pause(in Position a_position)

raises(InvalidPosition);

void resume(in Position a_position)

raises(InvalidPosition);

void stop(in Position a_position)

raises(InvalidPosition);

};

};

# Appendix C - Relationship to DAVIC work

The A/V Streams RFP deals with a very much different set of problems to those addressed by DAVIC’s DSM-CC. CORBA is used for all aspects of stream establishment and management with an emphasis on ensuring that heterogeneous devices talking to each other across the stream are compatible. The RFP response also addresses: multiple protocols, unipoint and multipoint stream topologies and many different types of flow, not just MPEG. It is aimed at the ‘off the shelf’ multimedia application programmer in general rather than those engineering  Video on Demand services in particular.

Nevertheless, there are areas of overlap with DAVIC’s DSM-CC User-User part. It is possible that a DAVIC stream could be managed as a flow within this architecture. In particular, an interface DAVIC_DSMCC_UU could be defined for DSM-CC User-User level operations. In particular, the DAVIC_DSMCC_UU could be used as the media controller for a VDev/FlowEndPoint and/or could be multiply inherited by a specialisatrion of FlowConnection i.e. interface davicFlowConnection : FlowConnection, DAVIC_DSMCC_UU;

# References

[1] 	Control and Management of A/V Streams, OMG RFP, telecom/96-08-01

[2]	Integrating Multimedia Streams into a Distributed Computing System, BJ Murphy and GE Mapp, ORL & Computer Laboratory, Cambridge.

[3]	IMA Recommended Practice, Multimedia Systems Services Part 1: Functional Specification, September 1994.

[4]	Measuring the Performance of Object-Oriented Components for High-speed Network Programming, Douglas C. Schmidt.

[5]	RTP: A Transport for Realtime Applications, RFC1889, March 1996.

[6]	OMG RFP5 Submission, Trading Object Service, orbos/96-05-06, Version 1.0.0, May 10, 1996.

[7]	J. Postel, Multipurpose Internet Mail Extensions (MIME) Part Two: Media Types, RFC 2046, November 1996.

[8]	C. Partridge, A Proposed Flow Specification, RFC 1363, September 1992.

[9]	TINA Object Definition Language (TINA-ODL) MANUAL, PBL01, TINA-C, (www.tinac.com/deliverable/deliverable.htm), July 1996

In the interests of backward compatability with the original specification, compliant implementation should also accept the flow name with “_QoS” appended, for example “audio_QoS” denotes a QoS structure associated with the flow “audio”.