# Evaluation of MECCANO Tools in Distance Learning Applications

Krzysztof Zieliński, Radosław Ruchała

{kz, ruchala} @cs.agh.edu.pl)
Department of Computer Science, University of Mining & Metallurgy
Al. Mickiewicza 30, Krakow, Poland
tel:+48 (12) 617 34 91, fax:+48 (12) 617 39 66

Abstract: The teleconferencing software tools developed under MECCANO project are presented and their usability in context of distance learning applications is evaluated. Three categories of tools are described: (i) conferencing session management, (ii) audio/video streams transmission and control, and (iii) presentation tools. Finally examples of their application under MECCANO Project are described.

Keywords: distance learning, videoconferencing, applications

## Introduction

The goal of this paper is to present the software tools developed under MECCANO project and their evaluation in context of distance learning applications. These tools have been designed having teleconferencing over ATM networks and others in mind. Despite of that they represent a grate potential that could be exploited in distance learning applications.

The paper is structured as follows. In Section 2 the distance learning tools requirements are shortly summarized. This creates a background for further considerations in Section 3, where the MECCANO teleconferencing tools are described. The non-A/V presentation tools are described in Section 4. Multimedia servers that provide video on demand and video editing capabilities are analysed in Section 5. Next in Section 6 the networking requirements are presented and the current status of MBone connectivity in Europe briefly evaluated. Section 7 contains description of examples of the MECCANO tools usage in distance learning. Finally in Section 8 the paper is concluded with a few remarks about the distance learning tools prospect of further development.

## Distance learning tools requirements

The MECCANO tools are oriented on teleconferencing but their functionality fulfils most of the requirements of distance learning systems. They address the most important part of such systems architecture related to video and audio distribution over broadband computer networks.

The determining factors of conferencing architecture are communication between (possibly large) groups of humans and real-time delivery of information. Early conferencing systems used a fan-out of data streams, e.g., one connection between each pair of participants, which means that the same information must cross some networks more than once. The Internet architecture uses the more efficient approach of multicasting the information to all participants.

Multimedia conferences require real-time delivery of at least the audio and video information streams used in the conference. In an ISDN context, fixed rate circuits are allocated for this purpose - whether their bandwidth is required at any particular instance or not. On the other hand, the traditional Internet service model (best effort) cannot make the necessary Quality of Service (QoS) available in congested networks. New service models are being defined in the Internet together with protocols to reserve capacity or prioritise traffic in a more flexible way than that available with circuit switching.

Figure 1. SDR v.2.5 – Main Window

The humans participating in a conference generally need to have a specific idea of the context in which the conference is happening, which can be formalized as a conference policy. Some conferences are essentially crowds gathered around an attraction, while others have very formal guidelines on who may take part (listen in) and who may speak at which point. In any case, initially the participants must find each other, i.e. establish communication relationships. During the conference, some conference control information is exchanged to implement a conference policy or at least to inform the participants of who is present.

In addition, security measures may be required to actually enforce the conference policy, e.g. to control who is listening and to authenticate contributions as purporting to originate from a specific person. In the Internet, there is little tendency to rely on the traditional security of distribution offered e.g. by the phone system. Instead, cryptographic methods are used for encryption and authentication, which need to be supported by additional conference set-up and control mechanisms.

Most of the presented requirements preserve full meaning in context of distance learning applications. Some of them, such as security and interactivity, or group communication are not so important.

## Teleconferencing tools

Teleconferencing tools may classified into three categories: (i) conferencing session management, (ii) audio/video streams transmission and control, (iii) presentation tools. The first two categories will investigated in more  details in this section.

SDR (Multicast Session Directory) v2.5 is a tool which assists the user in setting up and joining conferences. It is supports secure conference announcement. This means that it is possible to announce sessions to a specific group of people only. It is also possible to have the actual conferences encrypted as well.

Conferences which have been announced using SDR are listed in the tool’s main window (see Figure 1), rather like a TV listings guide. In the case of authentication, the digital signature is checked and the encrypted sessions are decrypted for the right group.

More information about each session can be obtained by clicking on its name in the list. A window will appear with further details about the event, security information and an invitation to join. When you join a conference using SDR the appropriate tools for that session will automatically be started up on the right addresses and with the right parameters. Alternatively, the user may decide to start up only a few of the tools. Not only the session announcement, but also the session itself may be private; this is ensured by enabling the tools to encrypt the data stream with a pass phrase. The pass phrase may be entered automatically, or through SDR.

With SDR anyone can create and advertise their own secure conferences, and invite other people to join if they wish. It provides a framework for setting up secure conference session announcements and automatically configures the relevant tools. Once a conference has been set up it will be announced to other users of SDR for a certain period of time (usually specified by the organizer of the conference). During this period other SDR users can join the session. If they have the appropriate keys they can check the authentication and decrypt the session. It is also possible to invite someone for a quick 5-minute consultation or a full meeting - like making a phone call.

Figure 2. Main RAT Window

SDR can receive and send encrypted and authenticated session announcements using technology such as DES, PGP and X.509. Figure 2 shows two authenticated sessions in the public sessions, which are signed using PGP and X509 technologies respectively. In the private sessions of Figure 2, four sessions are shown which are encrypted with DES, X509 and PGP and authenticated with PGP and X509. SDR can generate and send keys required for authentication and encryption. Eventually it will also be possible to use Smart Card technology to authenticate a session announcement or access encrypted information in a secured repository.

RAT (Robust Audio Tool) is unicast and multicast audio conferencing tool. RAT can be used for both point-to-point videoconferencing involving a direct link between two hosts or for multiparty conferencing with many participants via the Internet MBone.

The Main Window (see Figure 2) of RAT is split into three sections. On the left is a list of conference participants. Your own name will always appear at the top. Whenever you speak, your name will be highlighted. Clicking on the name of a participant (except yourself) will display a user information panel, giving various statistics for that user, such as reception statistics, which audio tool they are using, which version of the tool they are using, and so on .On the right of the Main RAT Window are volume/microphone controls. There are two sets of controls, the right set is for outgoing audio (microphone) and the left set is for incoming audio (headphones or speakers). Each set of controls consists of a mute button, a button for selecting the input/output device, a volume control and a power meter. The codecs supported by RAT are shown in the table:

| Name | Bit rate | Description |
| --- | --- | --- |
| L16 | 128 kbit/s | Linear PCM at 16 bits per sample |
| PCM | 64 kbit/s | -law PCM at 8 bits per sample (G711) |
| DVI | 32 kbit/s | Intel’s DVI ADPCM at 4 bits per sample |
| GSM | 13.2 kbit/s | EDSI Group Systeme Mobile CODEC |
| LPC | 5.8 kbit/s | Ron Zuckerman’s 10 pole LPC CODEC |

VIC is unicast and multicast videoconferencing tool. VIC can be used either for point-to-point videoconferencing involving a direct link between two hosts or for multiparty conferencing with many participants via the Internet MBone.

VIC consists of a Main Window, a Menu Window, and a number of smaller windows. In the following sections, the Main Window and the Menu Window will be discussed in greater detail. When nobody is transmitting video on the address/port that VIC has been started up on, the main window displays a “Waiting for video…” message. Otherwise, the Main VIC Window will automatically start displaying small thumbnail versions of the video images that are currently being transmitted (see Figure 3). The default size of the Main VIC Window can display three thumbnail images, so in order to see more thumbnail images you must enlarge the Main VIC Window.

Figure 3. Main VIC Window

Clicking the Help button at the bottom of the Main VIC Window brings up a window containing 7 bullet points explaining the basic operation of VIC. VIC provides the video coding indicated by the format argument for transmission. Format may be one of the following:

| jpeg | Motion JPEG | nv | Xerox PARC Network Video |
| --- | --- | --- | --- |
| mpeg | MPEG | nvm | uncompressed – nv's thresholding algorithm |
| h261 | Intra H.261 | cellb | Sun CellB |

Not all encodings are compatible with all frame grabbers. For instance, JPEG compression hardware is needed in order to source a JPEG stream (e.g., a DEC J300 or Parallax XVideo board).

## Presentation tools

Applications other than audio and video have evolved in Internet conferencing, ranging from shared text editors (NTE [han97]) to shared whiteboards (WB, DLB) and support for dynamic 3D objects (TeCo3D [mau99]). Such applications can be used to substitute for meeting aids in physical conferences (whiteboards, projectors) or replace visual and auditory cues that are lost in teleconferences (e.g. raising hands [mates], voting [mates, mpoll] and a speaker list); they can also enable new styles of joint work.

Non-A/V applications currently have vastly different design philosophies. This leads to a multitude of architectures and proprietary protocols both at the transport and at the application level. It is therefore a challenging task to combine them into a homogeneous teleconferencing toolset. Especially the development of generic services, like recording and playback of conferencing sessions, is currently impossible. These problems can be traced to two related areas: reliability and application layer protocols for non-A/V applications.

Many non-A/V applications have in common that the application protocol is about establishing and updating a shared state. Loss of information is often not acceptable, so some form of multicast reliability is required. However, the applications' requirements differ. Some applications just need a guarantee that each application data unit (ADU) eventually arrives; others require that the ordering of the ADUs transmitted by a single participant will be received in order. Applications might even demand that the total order of all ADUs send by all participants is preserved. Additionally applications have different requirements on the timeliness with which the packets need to be delivered.

Figure 4. Data flow in an Application Sharing scenario

The question of how to realise the reliability for different applications, given the wide range of reliability requirements, is one of the topics where work is still in progress in the IRTF research group on Reliable multicast [rmrg]. Other aspects of reliable multicast, which are not well understood, include how to provide congestion control in a multicast environment. As these issues are considered essential elements, standards track protocols are not expected before these can be solved.

Most conferencing solutions, which allow for collaboration between participants in a conference, provide either a single application distributed amongst the participants (Application Sharing) or a distributed data set on which all may work (Workspace Sharing).

Some conferencing tools allow additionally for text-based communication exchange, file transmission, acquisition of distributed information and other mechanisms to distribute or collect (non-audiovisual) information in the context of a running conference.

As shown in Figure 4, Application Sharing basically works by distributing the user interface of a single application. While that program is still running on a single machine, it may now be seen by multiple conference participants. If permitted by the user actually running the application, it may also be operated by several users - either simultaneously or sequentially, depending on the chosen floor control policy.

On the other hand, Workspace Sharing tools distribute a common data set among the participants of a conference, as illustrated in Figure 5. Each partner has a local representation of the shared workspace and may modify it at will (depending on the access control policy). By the exchange of messages, the sites try to achieve a consistent workspace state.

Figure 5. Data flow in a Workspace Sharing scenario

The shared workspaces described within this chapter allow several participants to modify simultaneously one or multiple documents in the context of a running conference (synchronous collaboration). There are other (asynchronous) systems [bscw][linkworks] defining a workspace as a set of documents (in terms of files created by an arbitrary non-distributed application) which can be consecutively modified by the members of a working group. In contrast to a mere distributed file system, these tools implement versioning and access control, support routing and workflow mechanisms, provide electronic signatures for individual documents, allow for approval and disapproval of modifications, keep track of deadlines and the status of documents and inform group members of any document changes they might be interested in.

In contrast to application-sharing environments, workspace-sharing tools distribute a common data set (the workspace) among the participants of a conference, allowing every partner to view (and possibly edit) his/her own part of a shared document independent of other users - if permitted. Every site has its own local representation of a shared workspace and may continue working with it even after other sites have become disconnected or left a conference. One of the primary duties of a workspace-sharing protocol is therefore to keep the individual (local) representations consistent with those at other sites and let them all converge to a common global shared workspace.

In principle, people participating in a workspace-sharing session may use different applications to work on the common data set. For the time being, however, workspaces are still application-specific: in order to be able to examine and modify a shared workspace the same tool has to be used at each site.

The lifetime of a shared workspace is determined by the period of at least one running instance of the corresponding application - i.e., as soon as all conference partners have left the conference, the workspace is lost. While some tools allow saving and (re) loading the contents of a workspace, there are no means for merging any changes applied to the saved data outside any session.

A new approach, which is currently being developed in the context of MECCANO, decouples the shared workspace from its applications and allows its objects to be deposited in an independent persistent store. A number of these stores then form a flexibly extensible federated workspace providing an environment for generic workspace sharing.

For MECCANO, workspace sharing is of higher importance than application sharing. Within the project, the following tools are used and/or developed:

Shared Whiteboard (wb and wbd) – originally developed at LBL as a test environment for scalable reliable multicast (SRM).

UCL Network Text Editor (NTE) – shared text editor designed for use on the MBone

Authoring-on-the-fly (AOFwb) – combination of a graphical whiteboard with a tool for tele-presentations.

Distributed Lecture Board (dlb) – basically, the Distributed Lecture Board dlb is an enhanced whiteboard tailored to the needs of synchronous teleteaching.

TeleCanvas – a Java-based shared whiteboard that tries to combine a hierarchical workspace model with a simple transaction model in order to realise a distributed undo/redo feature.

TeleStore – a new approach to generic workspace sharing is currently being developed in the context of MECCANO.

## Multimedia servers

Irrespective of their application area, all systems have a generally common architecture. The generic schematic of a Recorder/Player system is shown in Figure 6.

The different systems have different characteristics with respect to Figure 6. Many of the commercial Video-on-Demand systems such as IP/TV [iptv] from Cisco, Real Networks [realn] and Net-Show [nshow] from Microsoft, and the Oracle Video Server [ovs], concentrate only on the Player section. They usually have proprietary data formats in the data archive. In most cases they play through an Intranet, without multicast, to single players. In some the Client and Server are based on WWW clients and servers, with specific plug-ins for the media tools. These can operate over multicast only with a multicast proxy, since HTTP is defined only for unicast. However, many of these systems are designed for high performance, multiple sessions. They pay great attention to optimal use of disc storage - often by striping the data across the discs in the archive.

Figure 6. The generic architecture of a multimedia server system

For the rest of this paper we will consider only those recorder/players which work with multicast over the MBone. These normally use the generic architecture of Figure 6. At present all of them use the available MBone conferencing tools in the client role - though there are some variants.

The UCL development of MMCR [lam99] is typical of such recorder/players. MMCR is a system specifically designed for recording and playing back multicast multimedia conferences over the MBone. It has a client - server architecture (as shown in Fig. 8) and consists of the client User Interface and the server (which incorporates the playback, recording and browsing mechanisms); logical component independence simplifies development and component replication.

The ACC Scalable Video Distributed Architecture (SVDA) is another player/browser/recorder system, which illustrates the mechanisms that can be provided for large video stores. This system is designed to be able to process a significant number of simultaneous requests from a large library of multimedia data. At the highest level of abstraction, SVDA can be considered as consisting of three parts:

- The main server, responsible for serving user requests and managing the multimedia clips,

- VFS (Video File Server) proxies, covering video servers,

- AS (Archive Server) proxies, covering storage devices.

Two commercially available video-servers are used in this research: SunMedia Center and Oracle Video Server [ovs].

## Networking requirements

Multimedia applications require very often the multicast transport protocol such as for instance RTP for their efficient operation. Unfortunately the existing multicast routing protocols such as PIM,  MOSPF, or DVMPR  operates well in a single routing domain but exist problems with inter-domain multicast routing. In effect hierarchical multicast routing introduced a few months ago in European MBone is still not fully operational. The MBone activity in global scale is rather problematic and teleconferencing with partner spread around Europe may be effectively organized using the Reflector program. Reflector distributes multimedia streams between the registered partners using unicast connections. It means that the media stream must be replicated and the advantage of multicast data distribution is completely lost. It is important to point out that  usage of  Reflector is transparent for end-user applications.

The Internet provides reasonable quality. The meaning of reasonable, is difficult to determine; it depends on the performance desired. The key parameters are packets/sec, variability in packet arrival (jitter in ms), mean transit time of packets, stability of the Internet Connectivity of certain routes, availability of multicast in the Internet Provider. These parameters have different impact on the different services.

The normal Internet topology with multicast provision is called the MBone. This is a single topology and has no differentiation between different users. Even if all conferees have access to the MBone, there remains the question of whether the performance is adequate, and whether the demands on the bandwidth are too great. If all the above are in order, nothing needs to be done to compensate. Unfortunately, particularly the international MBone is very variable between countries, and they have different policies on how much bandwidth they provide - both nationally and internationally. Thus we often need to supplement normal MBone by unicast tunnels. If this is done, we have to be very careful not to upset the total MBone topology.

We must then construct reflectors at strategic points to ensure full multicast facilities. If we still have access to the MBone at any site, we must ensure, by route filtering and scoping, that the appropriate MBone topology is not disturbed.

The next parameter is throughput - in Kbps or packets/sec per audio stream; here the requirements depend on the media and quality desired. For audio tools like RAT, typical bandwidths needed are 8 - 32 Kbps; on good channels, with modern codecs, the higher of these give good quality audio. For video using tools like VIC, the corresponding rates are 50 Kbps - 3 Mbps. At the lower bandwidths, with slowly varying scenes, a few frames/sec can be achieved with QCIF, at the higher rates, full motion can be achieved at reasonable definition. The optimal media data rates for specific bandwidth depend also on the level and nature of channel errors. With audio, data arriving delayed or not at all is dropped. However, it is possible to provide deliberate redundancy in the talk-spurts to compensate for the loss. With video, similar errors will impact a whole frame. For this reason, while inter-frame coding will give much better performance at low error rates, it can be very inefficient at high ones. For this reason, most current implementations of tools like VIC do not currently use inter-frame coding; therefore higher data rates are needed for a given quality. We have done many measurements on data quality as a function of packet loss. We find loss rates of 15% still give reasonable quality of audio, with losses up to 40% tolerable with good use of redundancy. With no inter-frame coding, loss rates of up to 20% give quite tolerable video.

The next parameter is variability of delay, called jitter. In most of the audio tools, one can trade tolerance to jitter against delay. A jitter above a hundred ms in a replayed packet is so annoying, that this may be taken as an approximate cut-off point. If most packets have a variability of arrival between 50 and 150 ms, for example, then one can deliberately put in a delay of 150 ms in the replay buffer. Any packets arriving more than 150 ms late will be dropped. Similar considerations apply to video. If the delay is made too long, it is very annoying for fully interactive sessions; it is much less serious for one-way activities like lectures. A jitter above the cut-off point is exactly equivalent to packet loss.

The considerations for shared work-space tools are different. Here because the individual operations must be kept consistent, some form of Reliable multicast is normally used. Now it is the total rate of updates, packet loss, and connectivity that are the important factors. Overmuch packet loss will increase the traffic level on the network; the load is normally much less than that due to the audio and video.

## Practical experiences

MECCANO puts a lot of effort in practical evaluation of the tools which were developed during the project. One of such activities is project partners meeting, which is held via Internet using video conferencing tools. VIC and RAT are used for audio and video exchanging. All partners can see and hear each other discussing project management related issues like also exchanging technical knowledge covering subjects of activities.

Figure 7. UW Lectures – VIC

Additionally NTE application is used for making notes of the most important things, writing project actions which should be taken and keeping the discussion to follow the schedule. All the notes are distributed to the partners as minutes after the meeting. Such meeting are held regularly – about once for two weeks.

Beside that, both technical and management meetings play also an important role in checking the international MBone connectivity status, verifying network parameters required for video conferencing purposes.

Another very important activity is organizing transmissions of multimedia sessions to validate the use of the tools. A series of ten lectures from Institute of Social Studies Warsaw University, under the title “Social Change: Adaptation and Resistance”, were transmitted to the MBone. Main listeners of those seminars were people from School of Slavonic and East European Studies – a part of UCL London, but the lectures were also freely available to other participants. We used both RAT and VIC tools for transmitting audio and video between the sites – see Figure 7. Additionally NTE was used for behind the scenes technicians discussions, which did not disturb the lecture, but where also very important. All those tools were verified in practice and proved their usefulness. They are reliable, efficient, easy to get and use by the common user.

## Conclusions

The tools developed under  MECCANO project can be widely used in distance learning. It is already proved their usability but some requirements still need to be implemented. One of them is a tool for overheads or slides presentation by the lecturer. The common requirement is that efficient MBone connectivity and routing must be provided. This still seems to be a problem  across Dante  155Network.

## References

[bscw]	GMD, “BSCW Shared Workspace System”, http://bscw.gmd.de/

[han97]	Handley, M & Crowcroft, J, “Network Text Editor (NTE): A scalable shared text editor for the MBone”, Proceedings of ACM Sigcomm 97, Cannes, France,1997.

[iptv]	Cisco IP/TV, http://www.cisco.com/warp/public/732/net_enabled/iptv/index.shtml

[lam99]	Lambrinos, L et al., “The Multicast Multimedia Conference Recorder”, in Proceedings of the Seventh International Conference on Computer Communications and Networks, 12-15 October, Lafayette, Louisiana, USA.

[linkworks] Digital, Compaq, “LinkWorks an intelligent processor for mission-critical, document-based business processes”, http://www.digital.com/info/linkworks/

[mates]	Multimedia Assisted distributed Tele-Engineering Services (MATES), ESPRIT 20598 project, Information Technology in the 4th Framework Program of the European Community. http://www.cdt.luth.se/~mates/

[mau99]	Mauve, M, “TeCo3D - A 3D Telecooperation Application based on Java and VRML”. In Proc. of MMCN/SPIE'99, pp. 240-251, San Jose, USA, January 1999.

[mpoll]	Patrick, A, “User-Centred an MBone Videoconference Polling Tool”, March 11, 1998. http://debra.dgbt.doc.ca/mbone/mpoll/development/ MPoll application at http://www.merci.crc.doc.ca/mbone/mpoll/

[nshow]	Windows NT Server NetShow Services , 
NetShow player with Internet Explorer, Windows 95, and Windows NT
http://www.microsoft.com/Windows/windowsmedia/technologies/

[ovs]	Oracle Video Server, http://www.oracle.com/itv/ovs.html

[realn]	RealNetworks, http://www.real.com/

[rmrg]	IRTF Research Group on Reliable Multicast, http://www.east.isi.edu/RMRG/.

[sdr]	Session Directory (SDR), http://www-mice.cs.ucl.ac.uk/multimedia/software/sdr/